Prepared by

EA Engineering, Science, & Technology, Inc.

221 Sun Valley Blvd., Suite D

Lincoln, Nebraska 68528

Contract No. FA8903-10-R-9999-R139

Revised by

439th MSG/CEV

August 2015

Prepared for

HEADQUARTERS

Air Force Reserve Command

HQ AFRC/CEVQ

255 Richard Ray Boulevard

Robins Air Force Base, Georgia 31098-6137

StormWater Pollution

Prevention Plan

Westover Air Reserve Base

Massachusetts

EA Engineering, Science, & Technology, Inc., PBC

Lincoln, NE

TABLE OF CONTENTS

USECTIONU	UPAGE

1.	INTRODUCTION	1

1.1.	REGULATORY BACKGROUND	1

1.2.	PHYSICAL DESCRIPTION AND MISSION OF THE BASE	2

1.2.1.	Location and Surrounding Area	2

1.2.2.	439 AW Mission	4

1.3.	SCOPE AND OBJECTIVE	4

1.4.	STORM WATER POLLUTION PREVENTION PLAN HISTORY	4

2.	STORM WATER POLLUTION PREVENTION TEAM	5

2.1.	MEMBERS OF THE STORM WATER POLLUTION PREVENTION TEAM	5

2.2.	RESPONSIBILITIES OF THE STORM WATER POLLUTION PREVENTION TEAM MEMBERS	5

3.	ASSESSMENT OF STORM WATER CONTAMINATION POTENTIAL & SUMMARY OF POTENTIAL POLLUTANT SOURCES	6

3.1.	INDUSTRIAL AREA POINT (IAP) IDENTIFICATION AND SURVEY	6

3.1.1.	Identification of Industrial Activity Points (IAP)	6

3.1.2.	Survey of Industrial Activity Points	7

3.2.	BASE DRAINAGE AND WATERSHED CHARACTERIZATION	9

3.2.1.	Watershed Identification and Characterization	9

3.2.2.	Meteorological Summary	11

3.3.	MATERIAL INVENTORY	11

3.4.	PAST SPILLS AND LEAKS	11

3.5.	NON-STORM WATER DISCHARGES	14

3.5.1.	Identification of Non-Storm Water Discharges	14

3.5.2.	Identification and Elimination of Illicit Connections	14

3.6.	SUMMARY OF POTENTIAL STORM WATER CONTAMINATION BY INDUSTRIAL AREA	15

3.7.	THREATENED AND ENDANGERED SPECIES	18

3.8.	NATIONAL HISTORIC PLACES PROTECTION CERTIFICATION	18

4.	STORM WATER BEST MANAGEMENT PRACTICES (BMPS)	19

4.1.	GENERAL BASE-WIDE BMPS	19

4.1.1.	Good Housekeeping	19

4.1.2.	Preventive Maintenance	20

4.1.3.	Spill Prevention and Response	21

4.1.4.	General Pollution Prevention	22

4.1.5.	Employee Training	22

4.1.6.	Security Procedures	22

4.1.7.	Prohibition of Maintenance Activities Outside of Designated Areas	22

4.1.8.	BMPS That Need To Be Routinely Performed By Facility Personnel	23

4.1.9.	CSCE-Recommended BMPS	24

4.2.	STRUCTURAL BMPS	24

4.2.1.	Secondary Containment	24

4.2.2.	Oil Water Separators	24

4.2.3.	Change Drainage Destination to Sanitary Sewer	24

4.2.4.	Block Floor Drains / Disconnect Illicit Connections	25

4.2.5.	Install Shut-off Valves	25

4.2.6.	Crash Barriers/Bollards/Protective Piping	25

4.2.7.	Tank Overfill Alarms	25

4.3.	NON-STRUCTURAL BMPS	25

4.3.1.	Drip Pans/Secondary Containment Barrels and Pallets	25

4.3.2.	Spill Kits and Equipment Available	25

4.3.3.	Integrity Testing	26

4.3.4.	Cover Outdoor Storage/Transfer Area	26

4.3.5.	Change Location of HAZMATs	26

4.3.6.	Run-on Prevention	26

4.4.	OPERATIONAL BMPS	26

4.4.1.	POL Storage and Transfer BMPs	26

4.4.2.	Deicing BMPs	27

4.4.3.	Aircraft, Vehicle, and Equipment Washing BMPs	29

4.5.	BMPs FOR CONSTRUCTION PROJECTS AND OTHER AREAS WITH HIGH SOIL EROSION POTENTIAL	30

5.	EVALUATION OF STORM WATER POLLUTION PREVENTION PROCEDURES	31

5.1.	COMPREHENSIVE SITE COMPLIANCE EVALUATION (CSCE)	31

5.1.1.	Personnel to Perform Compliance Inspections	31

5.1.2.	Frequency and Extent of Inspection	32

5.1.3.	CSCE REPORT	32

5.2.	INSPECTION, SAMPLING, AND REPORTING REQUIREMENTS AND PROCEDURES	33

5.2.1.	STORM WATER SAMPLING AND SAMPLING DATA	34

5.3.	RECORDKEEPING	34

5.3.1.	Plan Revisions	35

6.	REFERENCES	36

UAPPENDICES

A	ABBREVIATIONS, ACRONYMS, AND GLOSSARY

B	WESTOVER ARB MULTI-SECTOR GENERAL PERMIT (2015)

C	< BUILDING INFO REDACTED FROM PUBLIC ACCESS >

D	CITY OF CHICOPEE SANITARY DISCHARGE PERMIT

E	Comprehensive Site Compliance Evaluation (CSCE) ReportS

F	ROUTINE IAP INSPECTION LOG & OUTFALL VISUAL ASSESSMENTS

G	ANNUAL REVIEW UPDATE CHECKLIST AND Record of DOCUMENT Review and Changes

H	WESTOVER ARB STORM WATER MONITORING DATA

I	PHOTOGRAPHIC RECORD OF STORM WATER OUTFALLS

J	LETTER OF DESIGNATION – AUTHORIZING REPRESENTATIVES OF STORM WATER PROGRAM

K	AIRCRAFT DEICING CHEMICAL QUANTITIES LOG

L                 AFFF PROTOCAL AND RELATED REGULATIONS

M                Substantially Identical Outfalls Assessment & Runoff Coefficient Calculations

N                 2013 E. Coli Source Assessment

LIST OF FIGURES

UFIGUREU	UPAGEU

< ALL FIGURES REDACTED FROM PUBLIC ACCESS >

LIST OF TABLES

TABLE                                                                                                                               	PAGE

Table 1. Outfall Characteristics	10

Table 2. Past Reportable Spills and Leaks	13

Table 3. Authorized Non-Storm Water Discharges	14

Table 4. Criteria Used to Assess Storm Water Contamination Potential	15

Table 5. List of Industrial Area Points and Non-Storm Water Survey Locations	17

Table 6. Threatened and Endangered County Species List for Hampden County, MA.	18

Table 7. Guidelines for Good Housekeeping	20

Table 8. Preventive Maintenance for Storm Water Pollution Prevention (AFTO 37-1-1)	21

Table 9. BMPs that Need to be Routinely Performed by Facility Personnel	23

Table 10. BMPs for Minimizing Deicing Runoff	28

Table 11. < REDACTED>	29

Table 12. Summary of Required Inspection, Sampling, and Reporting	33

# INTRODUCTION

United States Department of Defense (DoD) installations and personnel are required to comply with all federal, state, and local laws designed to protect the environment. Pursuant to this requirement, the United States Air Force (USAF) has developed Air Force Policy Directive (AFPD) 32-70, which prescribes general responsibilities, policies, and procedures to preserve, protect, and restore the quality of the environment. To implement this policy directive, Air Force Instruction (AFI) 32-7041, Water Quality Compliance, has been specifically developed to address compliance with a number of water quality issues, including storm water pollution prevention (SWPP). Under this and other USAF regulations, each major command is responsible for developing contingency plans and procedures for minimizing pollutant contributions to the environment through storm water contact and flow. This responsibility includes developing, maintaining, and implementing a written Storm Water Pollution Prevention Plan (SWPPP); such as this one for Westover Air Reserve Base (ARB).

## REGULATORY BACKGROUND

The Water Quality Act of 1987 amended the Clean Water Act to include the regulation of storm water discharges. In November 1990, the United States Environmental Protection Agency (EPA) published its phase one storm water regulations requiring large municipalities and specific industrial classes be covered under a National Pollutant Discharge Elimination System (NPDES) storm water permit. The EPA’s Region I is the NPDES control authority for the state of Massachusetts, and administers the storm water permitting program.

Westover ARB originally had an individual permit issued by EPA’s Region I (NPDES Permit No. MA0005444) to discharge storm water at Outfalls 001 and 002 until January 2000.  The remaining outfalls at the Base (Outfalls 003 through 009) were permitted by EPA Region I for coverage under the Multi-Sector General Permit (MSGP) published in the September 29, 1995, Federal Register (60 FR 50803).

In October 2000, the EPA reissued the NPDES Storm Water MSGP. The EPA evaluated the industry-specific requirements of the MSGP for re-issuing the permit. In January 2001 Westover ARB applied for coverage under 2000 MSGP and has included all outfalls (001 through 009) in the MSGP (Permit No. MAR05B973), which expired on 30 October 2005. The EPA grants automatic administrative continuance of the permit for current permitted facilities until a new MSGP has been issued, which the EPA does every five years.  The most recent MSGP permit was issued 4 June 2015.  Westover ARB has developed a storm water monitoring plan (with visual assessment and chemical analysis) to satisfy the EPA’s requirements.  Westover ARB is not required to meet numeric effluent limitations because such limitations are not contained in the MSGP for airport operations.

Coverage under the  MSGP requires the Base to certify that the storm water discharges do not have an impact on any threatened or endangered species regulated under the Endangered Species Act (ESA) or any historic properties regulated under the National Historical Preservation Act (NHPA). Certification statements have been added to the SWPPP in Section 3 to satisfy these requirements.

In lieu of numerical discharge limits, the MSGP requires the development of a SWPPP for controlling pollutant contributions to storm water at Westover ARB. Regulations determine the contents of the SWPPP. A SWPPP includes an assessment of current or potential sources of storm water pollution such as outside material storage, potential for spills and leaks, and aircraft deicing operations. Currently Westover ARB uses less than 100,000 gallons of deicing fluid annually and is not subject to the guidelines outlined for larger deicing operations.  The plan also identifies prohibited non-storm water discharges to the storm sewer system, including illicit connections.

This SWPPP includes an assessment of current or potential sources of storm water pollution such as: outside material storage, potential for spills and leaks, and other activities that may result in the discharge of contaminants to the storm water system. The SWPPP requires the identification of non-storm water discharges to the separate storm sewer system. This process involves meeting with Base personnel, reviewing sewer maps, and performing visual inspections and dye tests. Also, the SWPPP requires facilities to identify best management practices (BMPs) that are currently in place and to recommend any additional BMPs necessary to control storm water pollution. A BMP implementation schedule must be included in the plan to indicate compliance. Finally, the SWPPP requires each facility to establish a SWPP Team for implementing the plan, evaluating its effectiveness, and ensuring that it is revised and updated annually.

## PHYSICAL DESCRIPTION AND MISSION OF THE BASE

### Location and Surrounding Area

Westover ARB is composed of approximately 2,511 acres of land within the communities of Chicopee and Ludlow in the northern portion of Hampden County, Massachusetts. The Base is bordered by and in proximity to the Cities of Holyoke and Springfield, and the Towns of West Springfield, Granby, and South Hadley. Westover ARB is located 35 miles north of Hartford, Connecticut; and 90 miles west of Boston, Massachusetts. The Base is located in the Pioneer Valley Region, which encompasses 43 municipalities within Hampshire and Hampden Counties along the Connecticut River. The Base is situated approximately 2 miles east of the Connecticut River, and is traversed and/or bound by Cooley, Stony, and Willimansett Brooks. Figure 1 shows the location of Westover ARB in relation to Massachusetts and the surrounding region.

< RUNWAY DESCRIPTION REDACTED FROM PUBLIC ACCESS >

The activities and operations at Westover ARB are grouped by functional areas and land use categories, including aviation support, residential, commercial, industrial, medical, administrative, public facilities/recreation, and open space. The two primary land use categories are aviation support and industrial activities; which account for more than 50 percent of all facilities and square footage on Base.

Although the predominant land use surrounding the Base is residential, a large percentage of land is devoted to commercial and industrial uses, with 13 percent of the total land in the region consisting of cities and towns. Areas to the north and east of the Base consist mostly of rural communities with large agricultural and recreational uses; bordering the Base to the south and west is the town of Chicopee. According to the U.S. Census Bureau Hampden County had a population of 463,490 as of 2010.  <Base employee info redacted from public access.>

< FIGURES REDACTED FROM PUBLIC ACCESS >

Figure 1. Westover ARB and Surrounding Region

### 439 AW Mission

Westover ARB is home to the 439th Airlift Wing (439 AW) of the Air Force Reserve Command (AFRC). The 439 AW operates and maintains 8 C-5 aircrafts, representing 5 percent of United States’ total airlift capability. The 439 AW oversees the performance of medical evacuations and one flying squadron and 40 supporting units; responsible for the movement of troops, equipment, and supplies. The 439 AW is capable of providing air movement of troops, supplies, equipment, and medical patients. Airlift involves airdrop and combat off-load operations. Support units provide communications, engineering, logistical, medical and security support. The 439 AW also manages aircraft maintenance and all assigned Air Force real property, equipment, and supplies (439 AW 1996).

< TENANT DESCRIPTION REDACTED FROM PUBLIC ACCESS >

## SCOPE AND OBJECTIVE

This SWPPP is intended to be comprehensive, covering all applicable industrial operations at Westover ARB. The objective of this plan is to minimize the impact of storm water discharges from industrial sites to surface waters through the implementation of SWPP management practices. Section 2 of this document identifies the member(s) of the Westover ARB SWPP team and the responsibilities for implementing and maintaining this plan. Section 3 identifies areas at the Base that are industrial by regulatory definition and discusses the potential for storm water contamination from these areas. Controlling storm water pollution through use of BMPs is described in Section 4. Section 5 describes evaluation procedures required by the MSGP to ensure the continued effectiveness of the plan.

## STORM WATER POLLUTION PREVENTION PLAN HISTORY

This SWPPP was originally prepared for Westover ARB in December 1994. In March 2015, a Storm Water Pollution Prevention Comprehensive Site Compliance Evaluation (CSCE) of the Base was conducted. Any changes in industrial operations, along with findings and recommendations to further reduce storm water contamination, were recorded in the CSCE report, which can be found in Appendix E of this plan.

# STORM WATER POLLUTION PREVENTION TEAM

Each industry covered by the multi-sector permit must form and maintain a SWPP Team for the purpose of implementing all phases of this plan.

## MEMBERS OF THE STORM WATER POLLUTION PREVENTION TEAM

The ESOH Council members make up the Storm Water Team.

## RESPONSIBILITIES OF THE STORM WATER POLLUTION PREVENTION TEAM MEMBERS

The team is responsible for developing and revising the facility’s SWPPP as well as assisting the facility managers in maintaining control measures/BMPs and taking corrective actions where required.

# ASSESSMENT OF STORM WATER CONTAMINATION POTENTIAL & SUMMARY OF POTENTIAL POLLUTANT SOURCES

This section describes the initial assessment phase process of the SWPPP. It discusses the materials, data, and information that were gathered at Westover ARB and presents the methodology of how this information was used to originally prepare this plan. The primary purpose of this section is to locate the industrial areas at the Base exposed to storm water and identify the potential sources of storm water contamination from those areas.

The MSGP outlines tasks performed during the assessment phase process. This section is divided into seven subsections that describe each of the tasks performed and provides a summary of related findings at Westover ARB. The assessment process is outlined below along with the subsection number where each task is described:

3.1 Industrial Area Identification and Survey: Industrial areas were identified by reviewing Base environmental documents, plans, and utility drawings. These areas were then surveyed to identify areas of potential storm water contamination.

3.2 Base Drainage and Watershed Characterization: Base maps are provided that identify the location of industrial areas, outfalls, watershed boundaries, existing structural controls, surface water bodies, past spills and leaks, and non-storm water discharges.

3.3 Material Inventory: An inventory of materials that are potentially exposed to storm water and may contribute to contamination of storm water has been developed. In addition, this section discusses the possible pathways for contaminated storm water to enter the storm drainage system.

3.4 Past Spills and Leaks: Spills and leaks of hazardous or other materials that have occurred since 1997 were recorded, and any preventive actions used to prevent a reoccurrence are described.

3.5 Non-Storm Water Discharges: A variety of non-storm water discharges, including illicit connections, were identified through visual inspection, dye testing, review of utility schematics, and discussions with Base personnel.

3.6 Summary of Potential Storm Water Contamination: A summary of the results and findings from the assessment phase are presented in this section.

3.7 Threatened and Endangered Species: Lists threatened and endangered species in the geographic area of Westover ARB.

## INDUSTRIAL AREA POINT (IAP) IDENTIFICATION AND SURVEY

### Identification of Industrial Activity Points (IAP)

The first step of the assessment phase process was the identification of industrial activity points (IAPs), which are defined in federal storm water regulations as buildings or areas where certain industrial activities take place that could potentially contribute to storm water contamination. Through a review of Base documents, discussion with Base personnel, and visual inspection of the Base, each building or facility was placed into one of two categories: industrial (i.e., IAP) or non-industrial activity points.  Table 5. List of Industrial Area Points and Non-Storm Water Survey Locations presents the results of this IAP screening process. Some facilities at the Base were not included in the IAP screening process because they were obviously non-industrial. This list is updated annually based on the CSCE and a review of Base activities.

### Survey of Industrial Activity Points

Following the IAP identification process, a detailed survey was conducted at each IAP to identify the industrial activities performed, materials stored, BMPs in practice, and likelihood for non-storm water discharges from these locations. Results and findings from these surveys are presented in Subsection 3.6 of this plan. This review was facilitated by the use of an IAP survey form (Figure 2), which was completed for each IAP during the yearly visits. Appendix C contains all completed IAP survey forms for Westover ARB. The IAP survey form is comprised of the following sections:

- Inventory of Hazardous Materials Potentially Exposed to Storm Water: Lists and identifies any hazardous chemicals, petroleum products, or other materials that are present in substantial quantity that could potentially contact precipitation or runoff.

- Likelihood of Storm Water Contamination: Rates each IAP either high, medium, or low for its likelihood to be a storm water contamination source according to the criteria described in Section 3.6 of this plan.

- Description of Storm Water Entry Points and Ultimate Outfall Point: Describes areas where contaminated storm water could enter the storm drainage system and identifies the ultimate outfall point.

- Current BMPs in Place: Identifies the site-specific BMPs in existence at the time of the survey. (General BMPs, such as employee training, are not identified during this part of the site visit.)

- Suggested BMPs: Suggests any new BMPs that would mitigate storm water pollutant discharges.

- Other Information: Presents other related or interesting information.

| AFRC STORM WATER POLLUTION PREVENTION PLAN INDUSTRIAL ACTIVITY POINT (IAP) INSPECTION AT WESTOVER ARB mm/dd/yyyy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Number or Location: |  |  |  | Description: |  |  |  |  |  |
| Inventory of Hazardous Materials Potentially Exposed to Storm Water: |  |  |  |  |  |  |  |  |  |
| Likelihood of Storm Water Contamination: Explain: |  |  | High |  |  |  | Medium |  | Low |
| Description of Storm Water Entry Points and Ultimate Outfall Point: |  |  |  |  |  |  |  |  |  |
| Current BMPs in Place: |  |  |  |  | Suggested BMPs: |  |  |  |  |
| Other Information: |  |  |  |  |  |  |  |  |  |

Figure 2. Industrial Area Point (IAP) Survey Form (Yearly CSCE)

## BASE DRAINAGE AND WATERSHED CHARACTERIZATION

This subsection of the assessment phase characterizes the storm water drainage patterns and the watersheds by providing a visual description of the Base. Westover ARB also maintains a comprehensive Integrated Natural Resources Management Plan in addition to this SWPPP; which consists of wetland delineation and management plans (WARB 2008). This SWPPP contains two maps of Westover ARB that show the location of a variety of features and activities that are important with regard to storm water contamination potential. Figure 3 (back pocket) is an overall Base map that identifies the Base boundaries, the watershed boundaries, the industrial areas, and the storm water outfall points at the Base. Figure 4 (back pocket) is a larger scale insert that provides a more detailed look at the industrial areas and includes the location of the following areas:

Surface waters,

Vehicle and aircraft maintenance areas,

Fuel transfer and storage areas (for both ground vehicle and aircraft),

Oil/water separators,

Past spills and leaks,

Other industrial areas, and

Non-storm water discharges.

### Watershed Identification and Characterization

The topography of the Base is predominately flat, with elevations ranging from 230 to 240 feet above sea level.  < Base outfall location info redacted from public access. >  All of the outfalls eventually flow into the Connecticut River, located 2 miles west of the Base (USAF 1993a).

The Base finished constructing a submergent flow wetland in July 2002 to accept storm water from the northern side of the East Ramp that is connected to the airfield O/W separator Outfall 001. This project will further minimize the impact of deicing fluid on Cooley Brook. Constructed wetlands, used as a biological treatment method for wastewater containing organic and inorganic contaminants, have not been used frequently for treatment of aircraft deicing/anti-icing fluids to date. However, due to the readily biodegradable nature of glycol, use of a subsurface or reed-bed type constructed wetland could be expected to be successful in preventing large quantities of deicing fluids from directly discharging to surface waters.

< OUTFALL LOCATION DESCRIPTION REDACTED FROM PUBLIC ACCESS >.

Table 1. Outfall Characteristics

| OUTFALL | TOTAL DRAINAGE AREA (Acres) | IMPERVIOUS AREA (Acres) | PERCENT IMPERVIOUS (%) |
| --- | --- | --- | --- |
| 001 | 171.6 | 106.4 | 62 |
| 002 | 131.7 | 72.5 | 55 |
| 003 | 177.2 | 108.1 | 61 |
| 004 | 353.2 | 70.8 | 20 |
| 005a | 327.7 | 41.5 | 13 |
| 006 | 170.3 | 94.2 | 55 |
| 007a | 163.0 | 36.7 | 22 |
| 009a | 142.4 | 21.2 | 15 |
| 011a | 499 | 46.5 | 9 |
| Note: a Watershed is primarily non-industrial with the exception of runways |  |  |  |

During the 2013 CSCE, a review of the outfalls and associated watersheds was conducted by EA Engineering (see Appendix M).  It was determined that Westover ARB had multiple outfalls that met the requirements of substantially identical outfalls detailed in the 2008 MSGP.  The following outfalls have been determined to be substantially identical due to land use and industrial activities within each;

- Outfalls 002, 001

- Outfalls 003, 006

- Outfalls 005, 011

- Outfalls 007, 009

All outfalls will be sampled and inspected on a rotating basis as defined in the 2008 MSGP except for Outfall 003. WARB will no longer monitor this outfall due to its high level of off base drainage, minimal amount of Base industrial activities performed within the drainage basin, and the difficulty of reaching the outfall due to its location.  Outfall 006 will be inspected each quarter as a substantially identical to outfall 003.

Outfall 004 was found to not meet the criteria of substantially identical to any other outfalls and will continue to be inspected annually.

### Meteorological Summary

Westover ARB is located in Chicopee, Massachusetts, near Springfield in the south-central part of the state. The climate is typical of a New England maritime climate: cool, temperate and somewhat wet. The average July maximum temperature at Westover ARB is 85 degrees Fahrenheit (˚F); the average July minimum is 63˚F. The average January temperatures are 36˚F and 18˚F. The all-time record high temperature is 104˚F, and the record low temperature is -22˚F.

The average annual precipitation at Westover ARB is 43.9 inches. This precipitation is distributed very evenly, averaging between 3 and 4 inches in every month of the year. The highest monthly precipitation total on record is 18.68 inches. The maximum 24-hour rainfall event has been 7.55 inches. On average, rain events greater than one-half inch in 24 hours occur on 27 days per year. On average, close to 50 inches of snow falls annually at Westover.

## MATERIAL INVENTORY

For a material inventory identifying the industrial materials or chemicals at Westover ARB that could potentially be exposed to precipitation and contaminate storm water, refer to individual IAP survey forms in Appendix C. The IAP form provides an inventory of the types and quantities of hazardous materials (HAZMATs) present at each IAP, and identify the potential pathways of how these materials might enter the storm water drainage system. It also presents the ultimate outfall point for storm water from each IAP.

There are several pollutants/parameters within storm water at the Base that should be focused on. These parameters are biochemical oxygen demand (BOD), chemical oxygen demand (COD), total suspended solids (TSS), oil and grease, propylene glycol, potassium acetate, sodium formate, and various hazardous constituents of fuels (e.g., benzene, toluene, xylene, cyclohexane, ethylbenzene, and naphthalene). These contaminants can potentially enter storm water via spills and leaks, and illicit connections, and aircraft, apron, and roadway deicing activities during the deicing season from December to April. Appendix K includes an Aircraft Glycol Monitoring Log; details on this glycol monitoring log can be found in Section 4.4.2.Pollution prevention controls for these activities are presented in Section 4.

Some materials may be exposed to precipitation or runoff, but because of structural or non-structural BMPs will have minimal potential of entering the storm sewer system. A more detailed discussion of current and recommended BMPs is presented in Section 4.

## PAST SPILLS AND LEAKS

The MSGP requires documentation of all significant spills and leaks of oil, toxic, or hazardous pollutants that occurred during the previous 3 years. Significant spills include, but are not limited to, releases of oil or hazardous substances in excess of quantities that are reportable under Section 311 of the Clean Water Act (CWA) or Section 102 of the Comprehensive Environmental Response, Compensation, and Liability Act (CERCLA). Fuel spills in excess of 10 gallons are considered reportable in Massachusetts.

POL spills are the major storm water pollution concern at Westover ARB. Petroleum based fuels and oils have a variety of hazardous components (e.g., benzene, toluene, xylene, and naphthalene) that could have adverse impacts on receiving water quality in the event of an accidental discharge. Because high quantities of Jet Fuel-A (JAA) are used at the Base, it has the greatest potential for storm water contamination. Large quantities of JAA are transferred from the POL Complex to trucks used to transport the fuel to the flightline for aircraft refueling. In addition, fueling from a hydrant system is also performed at Westover ARB.

Table 2 provides a listing of reportable spills and leaks that have occurred in the 3 years prior to the date the SWPPP is updated. The spill log at the Base is maintained by the Fire Department (CEF). The log description must include the material spilled, quantity spilled, proximity to storm drainage access points, clean-up measures employed, and measures used to prevent a reoccurrence. Reporting requirements are discussed in Section 5 of this plan.

Table 2. Past Reportable Spills and Leaks

| DATE | LOCATION | SPILL DESCRIPTION | PREVENTATIVE ACTIONS |
| --- | --- | --- | --- |
| 2/12/14 | Building 7701 – Fuel Pit | 10 gallons of fuel were released into a fuel pit during maintenance activities being performed on the pipe. | The spill was contained, notifications were made, and a contractor was brought onsite to clean up the spill and remove any contaminated soils from the pit. The incident is closed. |
| 11/6/13 | Patriot Ave near building 2450 | 20 gallons of hydraulic fluid was released from a Sunbelt High Lift Equipment | MADEP was notified and issued RTN:19262. The area was cleaned by a spill contractor and all reports were completed. The incident is considered closed. |
| 9/26/13 | LIMA 5/6 | 20-25 gallons of JAA fuel was released from an F-15 aircraft due to over pressurization of fuel tanks. | MADEP issued a NOR on Sept. 27th, 2013 assigning RTN: 1-19230. On Sept. 26th oral approval was given to the WARB IRA Plan. The fire department cleaned up the spill using approved methods. This incident is considered closed. |
| 9/22/13 | ECHO 10 | 10 gallons of JAA fuel were released from an aircraft due to a faulty fuel valve. | MADEP was contacted and assigned RTN: 1-19225 to the incident. The area was cleaned and the outfall was monitored to make sure no fuel left he base. The incident is considered closed. |
| 8/20/13 | LIMA 6 | 25 gallons of JAA fuel was released from an F-15 fuel pod due to expansion of the fuel in the a full pod. | MADEP was notified and issued RTN: 1-19190. The area was cleaned and reports were submitted. The incident is considered closed. |
| 4/5/12 | Aircraft Parking Location E-12 | Notification was made to MADEP that approximately 100 gallons of JP-8 fueled spilled into a fueling pit with about 15 gallons overflowing onto the ramp. | The MADEP issued a Notice of Responsibility (NOR) on 4/5/12 assigning RTN 1-18686. The same day MADEP verbally approved the Immediate Response Action (IRA). The Westover ARb Fire department recovered fuel from the valve pit and placed down absorbent pads to clean up the spill. The site was inspected and has resulted in achievement of a “Permanent Solution”. The incident is considered closed. |
| - Source: WARB 2014. |  |  |  |

## NON-STORM WATER DISCHARGES

### Identification of Non-Storm Water Discharges

Non-storm water discharges can potentially originate from a variety of sources including illicit connections (Subsection 3.5.2. Identification and Elimination of Illicit Connections), deicing activities, and Fire Department suppression testing. A non-storm water discharge, as defined by the MSGP, is any discharge other than storm water runoff, snow melt runoff, and surface water runoff and drainage that enters the storm drainage system. Unless covered by a NPDES permit, non-storm water discharges are prohibited. However, there are a few exceptions authorized by the storm water regulations that allow certain non-storm water discharges to enter the storm sewer system, and these discharges are listed in Table 3.

Table 3. Authorized Non-Storm Water Discharges

AUTHORIZED NON-STORM WATER DISCHARGES

- Fire hydrant and fire safety system flushings

- Discharges from firefighting activities

- Foundation and footing drainage

- Flows from natural springs

- Exterior building wash downs (not containing detergents)

- Pavement wash downs where spills of hazardous materials have not occurred (not containing detergents)

- Potable water line flushings

- Lawn watering

- Air conditioning condensate

- Uncontaminated ground water

- Irrigation drainage

- Source: Section III.A.2 of the Multi-Sector General Permit

Currently fire-fighting foam, called Aqueous Film Forming Foam (AFFF), is being discharged to the sanitary sewer system and in grassy areas during testing of the fire suppression systems on Base.  To meet the requirements of the National Fire Protection Association (NFPA) and Air Force Standards, Westover ARB has developed an AFFF Readiness Protocol. This protocol, along with a description of relevant AFFF regulations is provided in Appendix L.

### Identification and Elimination of Illicit Connections

An illicit connection is a type of non-storm water discharge and occurs when indoor plumbing (e.g., floor drains, sinks, or trench drains) is connected to the storm sewer system rather than the sanitary sewer system. Determinations of flow drainage patterns and sewer connections are made by reviewing building utility schematics and Base sewer maps. However, if the drainage destinations remain unclear then visual inspections and/or dye testing is required.

Visual inspection of the storm sewer lines can often provide an indication of a non-storm water discharge. If there is flow present in the storm sewer during warm, dry weather, then it is often an indication of an illicit connection. A dye test can also be performed to aid in locating an illicit connection. The dye test consists of releasing dye into floor drains and sinks where illicit connections are suspected. Investigators, equipped with high-powered flashlights and communication devices, are then stationed at both the storm and sanitary sewer outlets to examine the discharge for discoloration.

In 1994, several IAPs were suspected of having illicit connections to the separate storm sewer system due to their proximity to storm sewer inlets, the age of the building, or the building manager’s description of flow patterns. To identify these illegal connections, an illicit connection survey was performed at Westover ARB in April 1994 as part of the initial SWPP site assessment. The survey included a review of building utility schematics and Base sewer maps, visual inspections, and dye testing.

In total, five dye tests were performed at three IAPs in 1994, and illicit connections were identified at Building Nos. 7000 (Fuel Systems Shop, Aircraft Inspection), 7084 (Fire Department), 1601 (Morale, Welfare, and Recreation), and 7711 (Refueler Truck Maintenance). Building 1601 was demolished in 2002 and Building 7000 no longer has illicit connections (see dye testing form in Appendix D). The illicit connections at Building Nos. 7084 and 7711 were identified through conversations with Base personnel and review of facility drawings. Floor drains in Building No. 7084 were re-piped to the sanitary sewer.  On 30 May 2006, the liquid in the 7711/7710 O/W separator was pumped out.  A mechanical plug was installed into the storm system line within the first manhole down-gradient of the separator.  This plug effectively cuts the separator off from the storm system.

## SUMMARY OF POTENTIAL STORM WATER CONTAMINATION BY INDUSTRIAL AREA

Upon completion of the IAP site surveys, the final storm water pollution assessment was performed. A determination of an IAP’s potential for storm water contamination was made based on the criteria shown in Table 4. Table 5 lists the IAPs that were surveyed and provides a summary of the storm water findings at each area. The second column from the right on the table rates each IAP (i.e., high, medium, or low) for storm water contamination potential according to the previously mentioned criteria. The relevant criterion applied to each IAP is identified in the last column of the table.

The objective of the SWPPP is to reduce storm water pollution by recommending that Westover ARB employ site-specific BMPs at each IAP with a “high” or “medium” rating, in order to reduce storm water contamination potential to a rating of “low.”

Table 4. Criteria Used to Assess Storm Water Contamination Potential

| POTENTIAL FOR STORM WATER CONTAMINATION | I.D. NO. | CRITERIA |
| --- | --- | --- |
|  | H-1 | • Frequent transfer of large quantities of POL or other hazardous materials, taking place near storm drainage access points not served by an adequate O/W separator or other positive means of controlling runoff. |
| High | H-2 | • Discharge or potential for discharge of high quantities of contamination to storm drainage access points. |
|  | H-3 | • Storage of hazardous material near storm drainage access points in a location where storm water precipitation is likely to directly contact that material (e.g., uncovered salt piles) and no adequate BMPs are present. |
|  | M-1 | • Transfer of POL or other hazardous materials taking place near storm drainage access points that are not served by spill kits, an adequate O/W separator, or other positive means of controlling runoff. |
|  | M-2 | • Discharge or potential for discharge of low/moderate quantities of contamination to storm drainage access points. |
| Medium | M-3 | • Storage of hazardous material away from storm drainage access points in a location where precipitation is likely to directly contact that material (e.g., uncovered salt piles) and no adequate BMPs are present. |
|  | M-4 | • Inadequate design, maintenance, or operation of the IAP O/W separator. |
|  | M-5 | • Storage of POL or other hazardous material not protected by adequate secondary containment. |
|  | L-1 | • Transfer or storage of POL or other hazardous materials with little or no potential for storm water contamination. |
|  | L-2 | • Little or no potential for discharge to storm drainage access points. |
| Low | L-3 | • Discharge to sanitary sewer. |
|  | L-4 | • Adequate maintenance or operation of the IAPs O/W separator. |
|  | L-5 | • Storage of POL or other hazardous material in a properly maintained or constructed secondary containment structure. |

Table 5. List of Industrial Area Points and Non-Storm Water Survey Locations

| Building | Description | Non-Storm Water Discharge or Illicit Connection? | Potential for Storm Water Contamination | Criteria I.D. Number |
| --- | --- | --- | --- | --- |
|  | < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | No | Low | L-1,2 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-5 |
|  |  | No | Low | L-5 |
|  |  | No | Low | L-5 |
|  |  | No | Low | L-1 |
|  |  | No | Low | L-1 |
|  |  | No | Low | L-5 |
|  |  | No | Low | L-1 |
|  |  | No | Low | L-3 |
|  |  | No | Medium | M-5 |
|  |  | No | Low | L-1 |
|  |  | No | Low | L-1 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-5 |
|  |  | No | Low | L-3 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-4,5 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-1 |
|  |  | No | Medium | M-2 |
|  |  | No | Low | L-2 |
|  |  | No | Low | L-2 |

## THREATENED AND ENDANGERED SPECIES

The 2008 MSGP requires Westover ARB to certify eligibility under this permit by ensuring that the storm water discharges, allowable non-storm water discharges, and discharge-related activities are not likely to jeopardize the continued existence of any species that are listed as endangered or threatened under the ESA. A certification statement and county-species list must be included in the SWPPP, along with supporting documentation on the eligibility determination.

The county-species list is found in Table 6.  The below species have not been found at Westover.

Table 6. Threatened and Endangered County Species List for Hampden County, MA.

| Group Name | Inverse Name | Scientific Name | Status |  |
| --- | --- | --- | --- | --- |
| Plants | Pogonia, Small Whorled | Isotria medeoloides | Threatened |  |
| Source: US Fish and Wildlife Service, 10/7/2011. |  |  |  |  |

The Westover ARB's Fish and Wildlife/Threatened and Endangered (F&W/T&E) Species Management Plan (SEA 1999) contains specific information regarding and also a list of the threatened and endangered species in the geographic area of Westover ARB. The U.S. Fish and Wildlife Service (USFWS) and the Massachusetts Natural History and Endangered Species Program (MNHESP) were contacted during development of the F&W/T&E Species Management Plan. The MNHESP completed several surveys in 1995 to confirm the presence or absence of federally listed endangered, threatened, or candidate plant and animal species at Westover ARB.  No federally listed plant or animal species were identified as inhabiting the Base during the surveys. Therefore, Criteria A of MSGP Section 1.2.3.6.3 has been satisfied, which certifies that, "…No endangered or threatened species or critical habitats are in the proximity to the facility or the point where authorized discharges reach the receiving waters…"

## NATIONAL HISTORIC PLACES PROTECTION CERTIFICATION

The 2008 MSGP requires Westover ARB to certify eligibility under this permit by ensuring that the storm water discharges, allowable non-storm water discharges, and discharge-related activities are not likely to affect a property that is either listed or eligible for listing on the National Register of Historic Places. A certification statement must be included in the SWPPP.

No facilities at Westover ARB are listed on the National Register Information System. As per the Natural and Cultural Resources Program Manager at Westover ARB, no facilities affected by storm water discharges or BMPs are eligible or are planned for listing on the National Register of Historic Places in the near future.  Therefore, Criteria A of MSGP Section 1.2.3.71. has been satisfied, which certifies that, "…the storm water discharges, allowable non-storm water discharges, and discharge-related activities do not affect a property that is listed or is eligible for listing on the National Register of Historic Places as maintained by the Secretary of the Interior…".

# STORM WATER BEST MANAGEMENT PRACTICES (BMPS)

The EPA guidance manual, Storm Water Management for Industrial Activities—Developing Pollution Prevention Plans and Best Management Practices [hereinafter referred to as the Storm Water Guidance Manual (SWGM)] uses the term “best management practices” (BMPs) for all SWPP measures, practices, and controls. This plan uses the term BMP in the same manner.

BMPs are measures used to prevent or mitigate pollution from any type of activity. BMPs are a very broad class of measures and may include processes, procedures, schedules of activities, prohibitions on practices, and other management practices to prevent or reduce water pollution. They may be inexpensive or costly. BMPs can be anything that prevents toxic, hazardous, or nuisance substances from entering storm water.

This section provides a detailed description of BMPs and is divided into two subsections related to the type of BMP and the areas in which they are applied. Site-specific BMPs that were identified and suggested during the annual compliance inspection can be found on the IAP inspection forms in Appendix C.

The BMPs suggested in this plan are the best, most cost effective way to correct potential storm water pollution problems, and they have been selected for the Base with the best available information. In the future, should information or industrial operations change, the Base may want to cancel plans to develop specific BMPs because a more effective BMP is available or because it is no longer necessary.

## GENERAL BASE-WIDE BMPS

There are numerous BMPs that apply generally to the Base as a whole. These Base-wide BMPs are discussed in this section.

### Good Housekeeping

Good housekeeping practices are designed to maintain a clean and orderly work environment. Often the most effective first step towards preventing pollution in storm water from industrial sites simply involves using good common sense to improve the facility’s basic housekeeping methods. Poor housekeeping can result in more waste being generated than necessary and an increased potential for storm water contamination. A clean and orderly work area reduces the possibility of hazards to shop personnel. Well-maintained and orderly material and chemical storage areas will reduce the possibility of storm water mixing with pollutants.

Currently, good housekeeping practices at the Base are a common sense BMP that are left up to the individual IAP managers. At most IAPs, good housekeeping practices are adequate and in some cases exceptional. To ensure good housekeeping is maintained at all IAPs at the Base, a specific set of guidelines was developed for managers to instruct their personnel to follow. The guidelines for good housekeeping practices are shown in Table 7. The guidelines are distributed to all area managers after semi-annual SWPP team meetings. In addition, the good housekeeping guideline is a topic covered in the annual SWPPP training course (see Subsection 4.1.5 for more details on SWPPP training.)

Table 7. Guidelines for Good Housekeeping

- Maintain dry, clean floors and ground surfaces by using brooms, shovels, vacuum cleaners, or cleaning machines (do not hose down floors with water)

- Use dry clean-up methods to collect spills, and ensure that spill cleanup procedures are understood by employees

- Store hazardous waste containers in the appropriate accumulation area

- Regularly pickup and dispose of garbage and non-hazardous waste material

- Make sure vehicles, equipment, and machinery are working properly

- Routinely inspect for leaks or conditions that could lead to discharge of chemicals or contact of storm water with raw materials, intermediate materials, waste materials, or products

- Remove debris from storm water catch basins

- Drain fluid from old parts prior to disposal

- Provide adequate aisle space to facilitate material transfer and easy access for inspections

- Store containers, drums, and bags away from direct traffic routes to prevent accidental spills

- Stack containers according to manufacturers’ instructions to avoid damaging the containers from improper weight distribution

- Store containers inside and on pallets or similar devices whenever possible to prevent corrosion of the containers which can result when containers come in contact with moisture on the ground

- Maintain an up-to-date inventory of materials to prevent overstocking and exceeding shelf-lives of materials

- Assign the responsibility of hazardous material inventory to a limited number of people who are trained to handle hazardous materials

- Perform washing, fueling, and maintenance activities inside or in designated areas

### Preventive Maintenance

Preventive maintenance involves the regular inspection, testing, and maintenance of storm water management devices (e.g., cleaning O/W separators, catch basins), plant equipment, and operational systems. These inspections uncover conditions such as cracks or slow leaks that could cause breakdowns or failures that result in discharges of chemicals to storm sewers and surface waters. The program prevents breakdowns and failures by adjustment, repair, or replacement of equipment.

Currently, Westover ARB has an extensive preventive maintenance program. It involves daily, weekly, monthly, and quarterly inspections, inventories, and reports at various industrial areas at the Base (e.g., hazardous waste accumulation points, the HAZMAT pharmacy, and the POL Complex). Visual inspections for leaks and conditions of drums, tanks and containers are currently conducted routinely as part of the preventive maintenance program at hazardous waste accumulation areas to meet RCRA regulatory requirements.  Although the preventive maintenance program was not necessarily developed with SWPP as its purpose, it does an excellent job of addressing potential storm water pollution problems. There are several preventive maintenance activities that are included with the Base’s program to specifically address items related to storm water, as required by Air Force Technical Orders and Operating Procedures. These activities are highlighted in Table 8 and are described in detail in Air Force Technical Order (AFTO) No. 37-1-1, General Operations and Inspection of Installed Fuel Storage and Dispensing Systems.

Table 8. Preventive Maintenance for Storm Water Pollution Prevention (AFTO 37-1-1)

- Inspect the aircraft deicing areas to ensure that the discharge of deicing fluid runoff is minimized as much as possible.

- Inspect O/W separators at least quarterly for build-up of oils, fuels, and sediments. If the inspection finds cleaning of certain separators is needed, then appropriate steps will be taken to clean the separators.

- Test pumps and piping at the POL Complex regularly for leaks or deterioration. Furthermore, replace seals in POL pumps periodically to prevent sudden leaks.

- Perform integrity testing of POL tanks and piping to prevent ruptures and leaks per AFTO No. 37-1-1.

- Inspect all secondary containment structures for cracks, breaks, holes, and deterioration per AFTO No. 37-1-1.

- Ensure all emergency shut-off switches, high level alarms, and other fuel storage and transfer-monitoring equipment are functioning properly by testing per AFTO No. 37-1-1.

### Spill Prevention and Response

Spills and leaks are together the largest industrial source of storm water pollutants, and, in most cases, are avoidable. Establishing standard operating procedures, such as safety and spill prevention procedures, along with proper employee training can reduce these accidental releases. Avoiding spills and leaks is preferable to cleaning them up after they occur, not only from an environmental standpoint but also because spills cause increased operating costs and lower productivity.

Spill kits are located at each IAP. The content of each spill kit is dependent on its purpose and location. For example, a spill kit in an area where fuel transfer occurs will need a non-sparking shovel, “speedy-dry” sorbent materials, sorbent pads, drain mats, etc.

The Base has combined three plans into one plan. The combined plan includes the SPCC, Facilities Response Plan, and the Hazardous Material Emergency Planning and Response Plan. This combined plan includes spill prevention and response procedures to meet federal, state, local, and USAF regulatory requirements for spill prevention and contingency planning and to meet SWPPP requirements. The combined plan addresses prevention of all spills including those that could contact storm water or flow to storm water drainage points. The plan also covers response capabilities and procedures for spills at the Base.

### General Pollution Prevention

The pollution prevention concept applies to more than storm water.  Before the storm water regulations were promulgated, waste minimization requirements existed for generators of hazardous waste, including Westover ARB. Furthermore, the USAF has on-going waste minimization and pollution prevention efforts. These efforts include source reduction studies, equipment changes, chemical substitutions, and logistical changes, which have led to reductions in HAZMATs used at the Base and reductions in the corresponding wastes generated. These changes reduce the potential for storm water pollution. Some examples of general pollution prevention at the Base include:

The Base uses fewer solvents for cleaning and degreasing. Self-contained solvent cleaning units maintained by a contractor allow for solvent recycling. This eliminates the storage of raw and waste solvents, thus lowering the potential for spills and contact with storm water.

The Base uses a Hazardous Material Pharmacy (HAZMART) system for its HAZMAT distribution. This system enables the Base to have a complete inventory of its HAZMATs and to store chemicals in one central location. The result is less waste, less risk of an uncontrolled spill or leak, and less cost.

Conversion from ethylene glycol to less hazardous propylene glycol for aircraft deicing, and from urea to potassium acetate and sodium formate for aircraft and apron deicing, are examples of chemical substitutions that lower storm water contamination toxicity.

Waste oil is managed as to prevent cross-contamination with other waste streams, which lowers disposal costs. Segregation is especially important with synthetic oils because they command a high resale value.

Waste antifreeze fluids from vehicles are recycled and reused, thus eliminating most of the waste.

### Employee Training

The Base SWPPP training consists of a PowerPoint presentation and/or a video. Training is provided to employees who work in areas where industrial materials or activities are exposed to storm water. Training covers BMPs used to achieve the effluent limits. Training is conducted annually.

### Security Procedures

- < REDACTED FROM PUBLIC ACCESS >

### Prohibition of Maintenance Activities Outside of Designated Areas

Certain activities related to vehicle, aircraft, and equipment maintenance must not be conducted outside of the areas designated for such activities. The areas that are designated for such activities are designed and managed to properly accommodate these activities without contributing to storm water contamination. These activities include the following:

Deicing aircraft;

Washing vehicle, aircraft, and equipment (Table 11 provides list of approved washracks);

Fueling vehicle, aircraft, and equipment;

Sanding, stripping, and painting vehicle, aircraft, and equipment; and

Storing chemicals outside.

Pesticide/ herbicide mixing must be conducted only in designated areas.

The prohibition of these activities outside of designated areas is communicated to Base personnel through SWPPP training. Furthermore, the visual inspections document evidence of these activities in non-designated areas.

### BMPS That Need To Be Routinely Performed By Facility Personnel

The list of BMPs that need to be implemented on a routine basis is shown in Table 9. Facility personnel should be trained and made aware of the implementation requirements for the routine, reoccurring BMPs in their areas.

Table 9. BMPs to be Routinely Performed by Facility Personnel

| IAP(s) | SUGGESTED BMP |
| --- | --- |
| < MILITARY BUILDING NAMES REDACTED FROM PUBLIC ACCESS > | - Perform routine visual inspections for leaks in long-term vehicle and equipment storage areas; use drip pans, where necessary, where leaks are identified. - Clean recent vehicle and equipment leaks in stained areas of the parking lot as quickly as possible. |
|  | - Use good housekeeping practices during salt loading/unloading operations. |
|  | - Sweep out the interior of dikes to prevent sediment from entering the O/W separator and storm sewer. - Perform visual inspections of the concrete containment dike. Seal cracks in the dike floor and walls as necessary. - Inspect refueler trucks regularly. |
|  | - Clean up small spills during fueling activities and visually inspect pumps to inspect for leaks. |
|  | - Perform visual inspections of the concrete containment dike. Seal cracks in the dike floor and walls as necessary. |
|  | - Park refueler trucks awaiting maintenance in the refueler parking area. - Store drip and collection pans containing residual oils away from drains. - Monitor analytical data at Outfall 002 to evaluate the effectiveness of BMPs. |
|  | - Perform routine visual inspection of water conservation pond for sediment and vegetation accumulation. Clean vegetation out of the water conservation pond as necessary to maintain liner integrity. |
|  | - Deice aircraft only in designated areas. - Inspect aircraft deicing areas quarterly during deicing operations to ensure deicing discharges are being minimized. - Inspect long term stored snow removal equipment and use drip pans as necessary. |
|  | - Use good housekeeping practices during soil loading/unloading operations. |

### CSCE-Recommended BMPS

The MSGP describes the implementation requirements for BMPs identified during the annual compliance inspection.  A current list of implemented BMP’s and recommended BMPs for each IAP can be found in Appendix C.

## STRUCTURAL BMPS

### Secondary Containment

Secondary containment generally consists of dikes or curbing around material storage and/or loading/unloading areas. The secondary containment dikes are impervious and capable of holding the entire contents of the largest single tank or container plus sufficient freeboard to allow for precipitation. Accumulated storm water is manually released or pumped out after storms or spills. This is done through a valve at the drainage point that is locked in the closed position and only opened by a trained attendant who verifies no contamination of the storm water has occurred prior to release. The attendant stays in the area until the valve is re-closed and locked.

Alternately, the storm water can be pumped out of a low spot, preferably by a sump, after verifying there is no contamination of the storm water. If the storm water is contaminated, it is properly disposed of based on the type and concentration of contaminants.

Another form of secondary containment is a secondarily contained aboveground tank (SCAT). A SCAT is a permanently double-walled or vaulted tank with an outer wall constructed of steel or concrete. The outer wall provides secondary containment for the inner tank and protects it from accidental damage. SCATs are used in place of the traditional aboveground storage tank (AST) inside a secondary containment dike.

### Oil Water Separators

O/W separators are control devices used to remove oil, grease, fuel, and other floatable materials from storm water. These materials are the most common and likely materials that could contaminate storm water at the Base. O/W separators act as treatment devices removing oils and fuels from intermittent flows of storm water, or they act as diversion and containment devices for minor spills of oils and fuels. The sizing and design of these units and the storage capacity for separated materials will determine to what extent the units can remove contaminants and contain minor spills.

### Change Drainage Destination to Sanitary Sewer

This is one of the more obvious methods for preventing storm water contamination. This method is used to eliminate non-storm water discharges and illicit connections. Identification of IAPs that have non-storm water discharges or illicit connections was conducted at the Base in 1994 and described in Subsection 3.5.2. For most of these cases, the best BMP was connection of the discharge or illicit connection to the sanitary sewer. Even better was stopping the non-storm water discharge if it was not needed, or moving the operation generating the wastewater to an area that drained to the sanitary sewer.

### Block Floor Drains / Disconnect Illicit Connections

Where non-storm water discharges can be permanently stopped and illicit connections (typically floor drains) are not needed, these connections are blocked or disconnected. Where the connection is needed for storm water drainage but transfer of materials occasionally occurs, the drain can be temporarily covered or blocked with a mat or plug.

### Install Shut-off Valves

Certain storm drains are located in areas where there is a high potential for storm water contamination. Permanent locked shut-off valves on these drains would prevent contamination from entering the storm drainage system. After a storm event, storm water is then checked for contamination and released by manually unlocking and opening the valve. An attendant must be present while releasing storm water. A clean-out access point may need to be installed prior to the valve in order to easily remove any spilled materials.

### Crash Barriers/Bollards/Protective Piping

Install barriers or bollards (hollow metal posts filled with concrete) around material storage and transfer areas to prevent accidental contact with vehicles, which could cause tanks or pipes to rupture. Also, protective piping or sheaths can be installed around fuel transfer hoses to protect hoses from accidental puncture.

### Tank Overfill Alarms

Tank overfill alarms are installed at aboveground and underground storage tanks to eliminate spills when filling tanks. An alarm sounds when liquid levels are approaching 100 percent capacity, thus signaling personnel to stop filling the tank.

## NON-STRUCTURAL BMPS

### Drip Pans/Secondary Containment Barrels and Pallets

Drip pans are used to catch drips from valves, pipes, etc. so that the materials or chemicals can be cleaned up easily or recycled before they can contaminate storm water. Although leaks and drips should be repaired and eliminated as part of a preventive maintenance program, drip pans provide a temporary solution where repair or replacement must be delayed. In addition, drip pans are an added safeguard when they are positioned beneath areas where leaks and drips may occur. Drip pans are also used where drips, leaks or spills are observed during visual inspections. Secondary containment for individual containers is provided by secondary containment barrels or special pallets that contain leaks and spills. These devices are used at key areas throughout the Base. Also, underground storage tank (UST) fill ports are outfitted with overfill prevention devices to prevent fuel spills.

### Spill Kits and Equipment Available

Spill kits (consisting of absorbents, pigs, etc.) are present wherever spills are likely to occur. Spill cleanup equipment (e.g., booms, shovels, vacuums, etc.) is present where large spills could occur. All IAPs and refueler trucks have spill kits and/or spill cleanup equipment. Refueler trucks have spill equipment on-board or have immediate access to spill equipment in the event of a spill. Spill kits are clearly labeled. The locations of spill kits and spill equipment and procedures for using them are covered in the combined plan.

### Integrity Testing

Integrity testing of tanks, piping, and valves should occur regularly. ASTs with adequate secondary containment should be integrity tested every five years. Piping and valves should be tested annually. Methods of integrity testing include:

Hydrostatic

Visual Inspection

X-ray/Gamma radiation

Ultrasonic

Acoustic Emissions

Shell Thickness

Helium

### Cover Outdoor Storage/Transfer Area

Covering an area where materials are stored helps prevent contact with storm water. The cover can be a permanent roof, temporary tarp or canopy, or spill pallet cover.

### Change Location of HAZMATs

HAZMATs that are used or stored in areas that could contaminate storm water should be moved, where feasible, inside a building or area that is better equipped or better suited to prevent storm water contamination. Also, HAZMATs that are no longer used in an area should be removed.

### Run-on Prevention

Run-on to all IAPs is prevented whenever possible; however, it is extremely important to prevent run-on to areas where fuel storage and transfer occur. Run-on is prevented by constructing diversion dikes and berms, storm water conveyances, and/or grading of land below these areas.

## OPERATIONAL BMPS

### POL Storage and Transfer BMPs

The greatest potential source of storm water contamination at Westover ARB is from POL storage and transfer sites. Aircraft and ground vehicle fuels (i.e., JAA, diesel, Mogas) are formulated from a variety of hazardous components that include benzene, cyclohexane, toluene, xylene, and naphthalene. These chemicals are highly toxic to fish and other biota in surface water. A variety of site-specific BMPs shall continue to be implemented for areas at the Base where POL storage and transfer occurs. These BMPs are listed below:

Drainage control measures to prevent or minimize storm water run-on to fuel issue and receiving areas. Measures include curbing, piping, gutters, sewers, grading, etc.;

Roof or cover over fueling areas;

O/W separators and/or positive means of controlling runoff from fuel transfer areas;

Secondary containment at the fuel storage areas with appropriate freeboard to allow for precipitation; and

Strong spill contingency and integrity testing plan.

<Infrastructure info redacted from public access.>

### Deicing BMPs

Westover ARB performs deicing/anti-icing operations on its aircraft and runways during snowstorms and freezing rain events. The application of deicing chemicals generates contaminated runoff that can enter the storm sewer system and severely impair surface water quality. Deicing compounds, because of their organic nature, exert a high BOD on receiving streams; depleting oxygen levels necessary to sustain aquatic life. In addition, these deicing compounds are also toxic to aquatic organisms. Other environmental impacts include glycol odors and glycol-contaminated surface water and groundwater systems.

The Base terminated use of urea during the winter of 1999-2000. Westover ARB’s urea usage for the 1996-1997 winter season was 20 tons, 28 tons in 1997-1998, and 28.5 tons in 1998-1999; however, approximately 95 percent of the urea used on-Base occurred at areas that do not drain directly to surface water. When urea breaks down, ammonia may be released, which is toxic in an aquatic environment.

The aprons, taxiways, and runways at the Base are currently deiced/anti-iced with potassium acetate (liquid) and sodium acetate (granular) in the winter. During the winter of 2014-2015, eighteen and a half tons of granular Sodium acetate, and 18,000 gallons of liquid Potassium Acetate were used.

The Base uses a non-triazole-based propylene glycol aircraft deicing fluid (PG-ADF), which is less toxic than PG-ADFs used in the past.  Deicing is conducted numerous times throughout the winter depending upon weather conditions, with the average deicing season being December to April. The Base keeps monthly records of the quantities of deicing chemicals used and conducts visual inspections monthly during the deicing season.  Appendix K contains a monthly quantities record of deicing chemicals used at the Base during the deicing season.  Propylene glycol for aircraft deicing is mixed at a 60/40 percent glycol/water ratio; the following list provides the mixed volumes of fluids used:  18,952 gallons during 2009-2010, 22,051 gallons during 2011-2012, 86,800 gallons during 2012-2013, 90,556 gallons during 2013-2014, and 75,678 gallons during 2014-2015.

The minimization and control of aircraft deicing fluid runoff is an important facet of SWPP.  Since 1991, the Base has reduced the amount and toxicity of aircraft deicing runoff that enters the storm sewer system through the use of BMPs.  Although the quantity of deicing chemicals used at the Base is low when compared to commercial airports, the Base needs to continue to use procedural and structural BMPs to minimize the amount of deicing runoff that enters surface waters. Westover ARB has developed a list of procedural BMPs for 439 MXG/AMXS Aircraft Maintenance Squadron Personnel to follow for minimizing deicing runoff during aircraft deicing operations. These procedures are described in Table 10 and are described in detail in AFTO No. 42C-1-2, Anti-Icing, Deicing, and Defrosting of Parked Aircraft.  Additionally, Aircraft Glycol Monitoring Logs, which include deicing date, location, amount of glycol used, A/C tail number and truck number, should be submitted to the CEV following each deicing event.

Table 10. BMPs for Minimizing Deicing Runoff

BMPs Currently Implemented for Runway Deicing:

Only areas necessary to flight operations are deiced using chemical methods, such as runways, taxiways, and flightline aprons.

Use of weather forecasting to postpone flight operations to minimize use of deicing chemical application.

Application of a pre-icing mixture to runway areas helps minimize the amount of deicing chemicals that need to be applied after icing occurs; less chemical is needed to prevent ice formation than it is to remove (melt) ice already there because once the ice builds up, it is difficult to remove.

Closely observe chemical usage on chemical application equipment to help prevent over application.

Train airfield deicing chemical applicators in proper application methods to prevent over use of deicing chemicals.

Pre-wetting of granular deicing chemicals to maximize effectiveness and minimize quantities needed.

Use of snow brooms (two active) and plows to remove ice instead of chemical applications.

Snow removal is continuous during a storm and accumulated snow is piled away from contact with deicing chemicals.

The prohibition of urea remains in force.

Suggested BMPs for Runway Deicing:

None

BMPs Currently Implemented for Aircraft Deicing:

- < AIRCRAFT INFO REDACTED FROM PUBLIC ACCESS >

Use of new Globemaster deicing trucks. Features include: enclosed application buckets, metered spray nozzles, more accurate spray nozzles, fluid heating capabilities, and enclosed cabs that allow personnel to apply deicing fluid more efficiently as back-spray and cold temperatures do not affect personnel’s performance.

Use of 60/40 deicing fluid mix; records kept of all chemical mixes used and applied.

- < AIRCRAFT INFO REDACTED FROM PUBLIC ACCESS >

Use of forced air deicing systems, airport traffic flow strategies, aircraft covers to minimize deicing chemical application.

- Suggested BMPs for Aircraft Deicing:

None.

BMPs Currently Implemented for Storm Water Treatment:

Constructed wetland and O/W separator serve most of the designated aircraft deicing locations.

Snow removal prior to application of deicing chemicals and stockpile of snow away from contact with deicing chemicals.

In addition to the runway deicing and snow removal, WARB has a Snow Management Plan that is updated yearly.  The plan provides direction for the collection of and stock piling of snow in areas away from drainage inlets to help contain contaminated melt waters.

### Aircraft, Vehicle, and Equipment Washing BMPs

The MSGP prohibits non-storm water discharges of vehicle, aircraft, and equipment wash water from entering surface waters of the United States. Therefore all wash water is discharged to the City of Chicopee Sanitary sewer system.  The Base obtained a Sanitary Discharge Permit from the City of Chicopee (Appendix D).  Wash waters contain oil and grease, suspended solids, and detergents, all of which would exhibit a high BOD on receiving waters. Wash water is considered a process wastewater and needs to be covered under a separate NPDES permit if discharged to a receiving stream (to be avoided whenever possible).

All vehicle, aircraft, and equipment washing must occur in designated areas where flow is directed to the sanitary sewer. Table 11 presents a list of approved wash racks at the Base that discharge to sanitary system.

Table 11. Approved Wash Racks at Westover ARB

< BUILDING DETAILS REDACTED FROM PUBLIC ACCESS>

When rinsing a vehicle with only water to remove mud (i.e., using no detergent or steam), use of a wash rack may not be necessary. A special area may be set up at the Base away from storm sewer inlets where runoff can flow over a grassy swale. A grassy swale is used to transport, filter, and remove sediments. It reduces runoff rates and allows for the infiltration of wash water. Requests for special rinsing areas are coordinated through the CEV.

## BMPs FOR CONSTRUCTION PROJECTS AND OTHER AREAS WITH HIGH SOIL EROSION POTENTIAL

Generally, the only areas of the Base that have a high potential for significant soil erosion are those areas disturbed by construction or facilities maintenance projects. Westover ARB has an extensive landscaping program that maintains good vegetative cover (grass, trees, and shrubs) throughout the Base. Furthermore, the soils and climate also contribute to maintenance of vegetation. Therefore, the primary need for sediment and erosion control will relate to construction or facilities maintenance projects.

Part 1.2.4.2 of the MSGP states: “You are not authorized for storm water discharges associated with construction activity unless in conjunction with mining activities, provided the applicable sector-specific requirements for construction storm water discharges as specified in Sector G and J of this permit are met.  Construction activity is defined in 40 CFR 122.26(b)(14)(x) or 40 CFR 122.26(b)(15).  These activities are covered by the EPA’s Construction General Permit.”

MSGP Part 2.1.2.5 Erosion and Sediment Controls states “You must stabilize exposed areas and contain runoff using structural and/or non-structural control measures to minimize onsite erosion and sedimentation, and the resulting discharge of pollutants.”

Detailed descriptions of erosion and sediment prevention BMPs can be found in the EPA manual Stormwater Management for Industrial Activities, otherwise referred to as SWGM.  Information on SWPP for construction activities can be found in the EPA guidance manual, Storm Water Management for Construction Activities—Developing Pollution Prevention Plans and Best Management Practices (USEPA 1992b).

# EVALUATION OF STORM WATER POLLUTION PREVENTION PROCEDURES

The overall effectiveness of the SWPPP shall be evaluated through a program of annual compliance inspections and accurate record keeping. If it is determined that the SWPPP is not effectively maintaining the quality of the storm water leaving the Base, the plan will need to be modified to correct these inadequacies. This subsection will discuss the specific requirements necessary to ensure the continued effectiveness of this plan.

## COMPREHENSIVE SITE COMPLIANCE EVALUATION (CSCE)

Compliance inspections are performed at the Base at least annually to (1) confirm the accuracy of the assessment of potential pollution sources contained in Section 3 of the plan, (2) determine the effectiveness of the plan, and (3) assess compliance with the terms and conditions of the MSGP. Annual compliance inspections are used not only to ensure that the risk factors associated with each industrial area are maintained at a low level through the use of BMPs, but also to ensure that other aspects of the SWPPP, such as the SWPP Team and annual training, continue to be maintained. In essence, the compliance inspections serve as a self-audit and result in recommendations for continued and improved storm water pollution prevention.

### Personnel to Perform Compliance Inspections

The compliance inspections are staffed by persons qualified to conduct a SWPPP investigation. These persons may be personnel either internal or external to Westover ARB, and should have knowledge of the goals and objectives of the SWPPP. In addition, these individuals should possess the following qualities:

Able to use the risk criteria presented in Table 4 to evaluate the current potential for industrial areas to contribute to the degradation of storm water.

Proficient in understanding how BMPs control storm water pollution from entering the environment, and capable of evaluating the effectiveness of BMPs and their current state of operation and maintenance. Personnel should also be able to recommend new or additional BMPs to supplement or replace current systems.

Adept at reviewing any applicable monitoring data to determine potential areas of the Base or activities within the Base that may be deleterious to the quality of the storm water.

Capable of reviewing changes in Base structure or operations that may have an adverse effect on the quality of storm water leaving the Base.

Competent at reviewing construction plans and erosion and sediment control documents to determine if erosion control methods and procedures are adequate to prevent sediment from entering storm water.

### Frequency and Extent of Inspection

Compliance inspections are performed at least annually or as soon as practicable after a major modification to the Base. The extent of the inspection includes all IAPs described in this plan for the 439 AW and its tenants, as well as other IAPs that may have been added to Base operations since the last review or implementation of this plan.

The first step of the compliance inspection is to determine the status of storm water pollution problems identified during the storm water compliance inspection of the prior year. Specifically, determine if illicit connections and other non-storm water discharges have been eliminated or if storm water BMPs have been implemented prior to the deadlines found in Section 4 of this plan.

The second step of the annual compliance inspection is to perform an assessment of the potential for storm water contamination from all new and existing IAPs at the Base. The criteria for risk analysis found in Table 4 should be used to determine the likelihood for storm water contamination from new and existing IAPs. Table 5 should then be revised to reflect these new determinations of storm water contamination potential. Personnel should visually inspect each IAP for evidence of, or the potential for, pollutants entering the drainage system. The HAZMAT storage practices at each IAP should be reviewed to determine their adequacy for preventing these materials from entering the storm drainage system either during storm events or during a spill.

The third step is to evaluate site-specific or Base-wide BMPs for their effectiveness in preventing pollutants from entering the environment. Inspections should determine if proper operation and maintenance of structural BMPs are being performed. Spill response equipment and plans should be inspected and reviewed to determine their effectiveness in preventing or mitigating spills that would contribute to storm water pollution. In addition, the inspectors should evaluate Base-wide, non-structural BMPs, such as the storm water training course, to ensure that they are being implemented. Any applicable monitoring data of storm water runoff should be reviewed to determine if these flows are a detriment to the quality of the storm water runoff.

### CSCE REPORT

The CSCE Contract procured by AFRC states that a report detailing the results of the inspection should be delivered to the Base within 2 weeks of the completion of the on-site inspection. The Base must implement any necessary changes to the plan to address recommendations supplied in the report. At a minimum, the following information should be included in the CSCE report:

Scope of the inspection;

Personnel making the inspection;

Date of the inspection;

Major observations relating to the implementation of the plan;

Description of the potential pollutant sources and the measures employed to control them;

Recommendations for additional BMPs, Base-wide or site-specific, to be implemented by the Base to control storm water pollution not already limited by existing methods;

Any incidents of noncompliance (e.g., spill events, etc.);

If the report does not identify any incidents of non-compliance, the report shall contain a certification that the facility is in compliance with the SWPPP and the permit; and

The report shall contain the following certification found in the multi-sector permit and be signed by the Base commander:

“I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to ensure that qualified personnel properly gathered and evaluated the information submitted. Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.”

The CSCE report must be retained as an addendum to the SWPPP for at least 3 years from the date of the inspection. The CSCE report for the Base can be found in Appendix E of this SWPPP.

The Base is required to submit an Annual report documenting the CSCE to the EPA using the form provided in MSGP Appendix I.  The Annual Report to EPA should be completed using the information gathered during the annual CSCE and submitted to the EPA within 45 days of the completion of the evaluation.

## INSPECTION, SAMPLING, AND REPORTING REQUIREMENTS AND PROCEDURES

Table 12 lists the major tasks to be performed, frequency, responsible party, and any reporting requirements.

Table 12. Summary of Required Inspection, Sampling, and Reporting

| Task | Frequency | Locations | Responsibility | Submittal Requirements |
| --- | --- | --- | --- | --- |
| Impaired Waters Sampling/Monitoring | Annually (Discontinued as of 2013 after assessing sampling results; Must Re-assess for new 2015 Permit.) | Outfalls that contribute to Stony Brook (Outfalls 005, 011, and 011a) | SW Program Manager via contracted lab | Submit to EPA |
| Visual Assessments at Outfalls | Quarterly | All outfalls on Base | SW Program Manager | NA |
| IAP Inspections | Quarterly and at least once during a wet weather event | All IAPs | SW Program Manager | NA |
| IAP Inspections (During Deicing Season) | Monthly during deicing season | All Affected Facilities | SW Program Manager | NA |
| CSCE | Annually | All IAPs and Outfalls | Contractor | NA |

Visual Assessments of outfalls will be conducted by the Environmental Flight. During these visual inspections, samples (wet weather) are collected at all outfalls.

Routine IAP inspections ensure that all of the elements of the plan are in place and working properly. Routine IAP inspections are not meant to be a comprehensive evaluation of the entire SWPPP—that is the function of the CSCE described in Section 5. Rather, they are meant to be a routine observation of the facility to identify conditions that may give rise to contamination of storm water runoff with pollutants from the facility. The routine IAP inspection is also a way to confirm that BMPs are in place and working properly. Visual inspections for leaks and conditions of drums, tanks and containers are currently conducted routinely as part of the preventive maintenance program at hazardous waste accumulation areas to meet RCRA regulatory requirements.  Environmental Engineering staff members conduct the routine IAP inspections quarterly, document the inspections, and maintain the records in a Microsoft Excel spreadsheet.

| TEMPLATE IAP INSPECTION FORM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IAP LOCATION (BUILDING NO.) | Suggested BMPs | Check (x) if BMP needs to be performed routinely | Notes/ Comments | Date & Time | Date & Time | Date & Time |

Figure 5. Template IAP Inspection Log

### STORM WATER SAMPLING AND SAMPLING DATA

The MADEP has classified Stony Brook as an Impaired Waterway with the pollutants of concern being TSS and Escherichia coli (E. coli).  During the first sampling event in April 2009 the TSS levels were found to be either non-detect or below reportable limits at the regulated outfalls. Pursuant to Part 6.2.4.2 of the MSGP, the Base is no longer required to monitor TSS.  The Base conducted  E. coli. monitoring from 2009 until 2012 at Outfalls 005, 011, 011a.  During the 2013 CSCE EA Engineering conducted a review of available sampling data collected by Westover ARB and historical monitoring data from Stony Brook (See Appendix N).   It was determined that the Base was not a significant source of E. coli and E. coli colonies present in the outfalls sampled was under the primary use recreational contact limit of 126 colonies per 100ml and below the calculated geometric mean of 290 cfu/100ml  presented in the Connecticut River Watershed 2003 WQ Assessment Report (CONN 2013). The findings and determination was submitted to the EPA with the 2013 Discharge Monitoring Report (DMR) which can be found in the Environmental Engineering Office at the Base. The MSGP requires those airports that use more than 100,000 gallons of glycol-based deicing chemicals and/or 100 tons or more of urea on an average annual basis to conduct storm water monitoring. Westover ARB does not use deicing chemicals in these quantities; therefore, Westover ARB was not required to conduct storm water monitoring in the associated outfalls for the remainder of the 2008 MSGP coverage.  Sampling for E.coli must be conducted for each new issuance of the MSGP.  Thus sampling will be resumed pursuant to the 2015 MSGP and Westover’s contribution to the presence of E. coli will be re-assessed.

Historical voluntary sampling results for TSS, pH, and oil and grease at the Outfalls are available.

## RECORDKEEPING

Recordkeeping associated with this SWPPP documents the Base’s efforts to minimize and control discharges to the storm water system. Records are maintained for the following major documentation areas:

Records of spills, leaks, and other discharges;

Records of inspections and maintenance activities;

Any applicable monitoring data; and

Other supporting data.

The Base shall retain this SWPPP for 3 years after coverage under this permit expires. The Base shall retain all records of all monitoring data, copies of all reports required by the MSGP, and all records of all data used to complete the NOI for at least 3 years from the date of measurement, report, or application.

### Plan Revisions

Westover ARB will amend this plan whenever there is a change in design, construction, operation, or maintenance which has a significant effect on the potential for the discharge of pollutants to the waters of the state of Massachusetts, or if the SWPPP proves to be ineffective at eliminating or significantly minimizing pollutants from sources identified in this plan, or in otherwise achieving the general objectives of controlling pollutants in storm water discharges associated with industrial activity. A record or log of all changes to the plan will be maintained (Appendix G).

# REFERENCES

| 439 AW 1996 | 439th Airlift Wing (AW). 1996. United States Air Force Fact Sheet. Westover Air Reserve Base (ARB). 1996. |
| --- | --- |
| SEA 1999 CONN 2003 | Final Fish and Wildlife/Threatened and Endangered Species Management Plan for Westover Air Reserve Base, Massachusetts, July 1999. Science and Engineering Associates, Inc. Connecticut River Watershed 2003 Water Quality Assessment Report. Commonwealth of Massachusetts, 2003. |
| Thompson 1993 | Storm Water Permit Manual, Volume One. Thompson Publishing Group, Washington, D.C., May 1993. |
| USAF 1991 | Revised Group Permit Application for Storm Water Discharges for U.S. Air Force. U.S. Air Force Headquarters, Washington D.C., 21 September 1991. |
| USAF 1992 | Composite Utility Systems Schematics for Westover Air Reserve Base. U.S. Air Force, Chicopee, MA, December 1992. |
| USAF 1993a | Storm Water Report for Westover Air Reserve Base. U.S. Air Force, Chicopee, MA, May 1993. |
| USAF 1993b | Final Pollution Prevention Assessment and Feasibility Report for Westover Air Reserve Base. U.S. Air Force, Chicopee, MA, August 1993. |
| USAF 1993c | Hazardous Waste Management Plan for Westover Air Reserve Base. U.S. Air Force, Chicopee, MA, December 1993 or Current Years Addition. |
| USAF 1993d | Dye Test Investigation, Hangars 1, 3, 5, 7, and 9 for Westover Air Reserve Base. U.S. Air Force, Chicopee, MA, July 1993. |
| USEPA 1992a | Storm Water Management for Industrial Activities, Developing Pollution Plans and Best Management Practices. U.S. Environmental Protection Agency, Office of Water (WH-547), EPA 832-R-92-006, September 1992. |
| USEPA 1992b | Storm Water Management for Construction Activities, Developing Pollution Plans and Best Management Practices. U.S. Environmental Protection Agency, Office of Water (WH-547), EPA 832-R-92-005, September 1992. |
| WARB 2008 | Westover Air Reserve Base (WARB). 2008. Integrated Natural Resources Management Plan. Prepared for Westover ARB by Stone and Webster Environmental Technology and Services and LAW Engineering and Environmental Services. Chicopee, Massachusetts. 2008 or Current Years Addition. |
| WARB 1997 | Westover Air Reserve Base (WARB). 1997. Wetland Identification and Delineation Report. Prepared by Science & Engineering Associates, Inc. (SEA). August 1997 or Current Years Addition. |
| WARB 1999a | Westover Air Reserve Base (ARB). 1999. Westover ARB webpage. http://www.afres.af.mil/units/439AW/Default.htm. July 1999. |
| WARB 1999b | Westover Air Reserve Base (ARB). 1999. Westover ARB Fire Department Run Tracking System. April 1994 and April 1999. |
| WARB 2002 | Westover Air Reserve Base (ARB). 2000. Phone interview with Public Affairs personnel. October 21, 2002. |
| WARB 2011 | Westover Air Reserve Base (WARB). 2011. Hazardous Material (HazMat) Emergency Planning and Response Plan (Facility Response Plan and SPCC Plan). Prepared by ECATS. April 2011 or Current Years Addition. |

APPENDIX A

ABBREVIATIONS, ACRONYMS, AND GLOSSARY

Abbreviations and Acronyms

AFFF	Aqueous Film-Forming Foam

AFI	Air Force Instruction

AFPD	Air Force Policy Directive

AFRC	Air Force Reserve Command

AFTO	Air Force Technical Order

ARB	Air Reserve Base

AST	Aboveground Storage Tank

AW	Airlift Wing

BCE	Base Civil Engineer

BLDG	Building

BMP	Best Management Practices

BOD	Biochemical Oxygen Demand

CE	Civil engineering

CEF	Fire Department

CERCLA	Comprehensive Environmental Response, Compensation, and Liability Act

CEV	Civil environmental engineer

COD	Chemical Oxygen Demand

CSCE	Comprehensive Site Compliance Evaluation

CWA	Clean Water Act

DMR	Discharge Monitoring Report

DoD	Department of Defense

EA	EA Engineering, Science, & Technology, Inc., PBC

EPA	United States Environmental Protection Agency

ESA	Endangered Species Act

ESOH	Environmental Safety and Occupational Health Committee

F&W/T&E	Fish and Wildlife/Threatened and Endangered

GAL	Gallon

HAZMART	hazardous materials pharmacy

HAZMAT	Hazardous Materials

HQ	headquarters

JAA	Jet Fuel A

MADEP	Massachusetts Department of Environmental Protection

MA ARNG	Massachusetts Army National Guard

MH	Manhole

MNHESP	Massachusetts Natural History and Endangered Species Program

MOGAS	Motor Vehicle Gasoline (Unleaded)

MSGP	Multi-Sector General Permit

NHPA	National Historical Preservation Act

NOI	Notice of Intent

NPDES	National Pollutant Discharge Elimination System

O/W	oil/Water

PG-ADF	Propylene Glycol Aircraft Deicing Fluid

POL	Petroleum, Oils, and Lubricant

RCRA	Resource Conservation and Recovery Act

RIAP	Regulated industrial activity point

RIDS	Runway Ice Detection System

SCAT	Secondarily Contained Aboveground Tank

SWGM	Storm Water Guidance Manual

SWPPP	Storm Water Pollution Prevention Plan

SWPP	Storm Water Pollution Prevention

TKN	Total Kjeldahl Nitrogen

TSS	Total Suspended Solids

USAF	United States Air Force

USFWS	United States Fish and Wildlife Service

USMC	United States Marine Corps

UST	Underground Storage Tank

WMDC	Westover Metropolitan Development Corporation

Glossary

BEST MANAGEMENT PRACTICES (BMPs)—BMPs are schedules of activities, prohibitions of practices, maintenance procedures, and other management practices to prevent or reduce the pollution of surface waters.  In essence, they are any method, short of actual treatment, used to limit storm water pollution.

BOLLARD—A concrete or metal pipe placed vertically in the ground to protect equipment (e.g., transformers, ASTs, etc.) from collisions with ground-based vehicles.

CERCLA—The Comprehensive Environmental Response, Compensation, and Liability Act of 1980, as amended by the Superfund Amendments and Reauthorization Act of 1986.

DISCHARGE OR SPILL—An act or omission by which oil, hazardous substances, or other substances in harmful quantities (see definition) are spilled, leaked, pumped, poured, emitted, entered, or dumped onto or into waters in the State or by which those substances are deposited where, unless controlled or removed, they may drain, seep, run, or otherwise enter water in the State, whether done accidentally or intentionally.  The term shall not include any discharge that is authorized by a permit issued pursuant to Federal or State law.  Discharge or spill also means threat of discharge or spill.

ENVIRONMENT—As defined by Section 101(8) of CERCLA means the navigable waters of the U.S., and any other surface water, ground water, drinking water supply, land surface or subsurface strata, or ambient air within the U.S. or under the jurisdiction of the U.S.

ENVIRONMENTAL Safety and Occupational Health COMMITTEE (ESOH)—A committee chaired by the 439th Airlift Wing Commander and comprised of the Unit Chiefs of major offices.  The ESOH is tasked under AFI 32-7005 to review Base environmental policy, facilitate coordination of environmental programs, review the spill prevention and response plan, hazardous waste management plan, and review and coordinate the installation environmental monitoring program.

FLIGHTLINE APRON—This area is used to prepare aircraft for departure or park aircraft upon arrival.  Deicing and fuel transfer activities occur here, but no maintenance.

FUEL—A flammable combustible liquid of any kind, including, but not limited to gasoline, JP-8, diesel, and naphtha.  Fuel spills are defined as follows:  Class I, or minor fuel spills, involve an area less than two feet in any dimension and are not of a continuing nature.  Class II, or medium fuel spills, are larger than Class I spills, and are less than 10 feet in any dimension, or not over 50 square feet in area, and are not of a continuing nature.  Class III, or major fuel spills, include spills that are over 10 feet in any one dimension, over 50 square feet in total area, or are of a continuing nature.

HAZARDOUS MATERIALS (HAZMAT)—Many definitions and descriptive names are used for the term hazardous materials.  For purposes of this SWPPP, hazardous materials include hazardous substances, petroleum products, natural/synthetic gas, acutely toxic chemicals, and other toxic chemicals in the solid or liquid form only.

HAZARDOUS SUBSTANCE—Any substance designated as such by the Administrator of the EPA pursuant to CERCLA (40 CFR 302); regulated pursuant to Section 311 of the CWA, or designated as such by the State.

HAZARDOUS WASTE—Any solid waste identified or listed as a hazardous waste by the Administrator of the EPA in 40 CFR 261, Subparts C and D.  40 CFR 261 Subpart C identifies the characteristics of hazardous wastes and 40 CFR 261 Subpart D identifies those wastes listed as hazardous.

ILLICIT CONNECTION—Indoor plumbing that is connected to the separate storm sewer system and not the sanitary system.

MOGAS—Fuel used predominantly in piston engines, such as small trucks, passenger cars, and other ground-based vehicles.  Mogas is usually unleaded gasoline and not diesel fuel.

MUNICIPAL SEPARATE STORM SEWER SYSTEM (MS4s)—MS4s are separate storm sewers that are either owned or located in a city with a population of 100,000 or more persons.

NATIONAL POLLUTANT DISCHARGE ELIMINATION SYSTEM (NPDES) PERMIT—The program developed by EPA in response to the Clean Water Act that is designated to control pollutants entering waters of the U.S., either from point or non-point (i.e., diffuse) sources.

NON-STORM WATER DISCHARGE—Any discharge other than storm water runoff, snow melt runoff, and surface water runoff that enters the storm drainage system.

OUTFALL—The point where storm water enters a natural waterway, enters a municipal separate storm sewer system, or exits installation boundaries.

REGULATED INDUSTRIAL ACTIVITY POINTS (RIAPs)—Buildings or areas where certain industrial activities take place that could potentially contribute to storm water contamination.

RELEASE—As defined by Section 101(22) of CERCLA, means any spilling, leaking, pumping, pouring, emitting, emptying, discharging, injecting, escaping, leaching, dumping, or disposing into the environment, but excludes:  (a) any release which results in exposure to persons solely within a workplace; (b) emissions from engine exhaust; (c) release of source, byproduct, or special nuclear material from a nuclear incident; (d) the normal application of fertilizer; and (e) activities authorized by the State.

REPORTABLE QUANTITY (RQ)—As defined in 40 CFR 117.1(a) means the quantity of a hazardous substance determined to be harmful by EPA.  A discharge in the environment equal to, or greater than, the RQ must be reported to the NRC and other appropriate State and Federal agencies.  The list of hazardous substances and their respective reportable quantities is found in 40 CFR 302 and is periodically updated and republished in the Federal Register.  Until amended by regulation under CERCLA Section 102(a), CERCLA Section 102(b) establishes a reportable quantity of one pound for hazardous substances other than those hazardous substances with RQs established under 33 USC SS 1321, Section 311 of the CWA; for these latter substances, Section 102(b) adopts the established RQs.

REPORTABLE SPILL—Discharge or release into the environment that requires reporting to State and Federal agencies based on the amount spilled (reportable quantity); danger or threat to environmental/public health; or requirements of AFI 32-7005.  Environmental Management shall notify appropriate State and Federal agencies of all reportable spills.  Notification shall be coordinated with the Incident Commander, On-Scene Coordinator, Staff Judge Advocate, and Public Affairs.  NOTE:  This definition does not apply to the Base spill notification requirements of this plan.

SEPARATE STORM SEWER SYSTEM—A storm water conveyance used solely to transport storm water runoff and drainage.

SIGNIFICANT SPILL—These include, but are limited to, releases of oil or hazardous substances in excess of quantities that are reportable under Section 311 of the CWA, or Section 102 of CERCLA.  Significant spills can also include releases of oil or hazardous substances that are not in excess of reporting requirements and releases of materials that are not classified as an oil or a hazardous substance.  The 439 AW considers significant spills to be any hazardous spill or leak that is reported to EPA Region I.

SPILL—See Discharge.

STORM WATER POLLUTION PREVENTION TEAM—An individual or individuals at the installation that are responsible for assisting the Base Commander with the implementation, maintenance, and revision of the SWPPP.

APPENDIX B

WESTOVER ARB MULTI-SECTOR GENERAL PERMIT (2015)

[MSGP is a national general permit which can be found on EPA’s website.]

APPENDIX C

WESTOVER ARB IAP SURVEY FORMS

< BUILDING INFO REDACTED FROM PUBLIC ACCESS >

APPENDIX D

CITY OF CHICOPEE SANITARY DISCHARGE PERMIT

(See 439 MSG/CEV Environmental Office to view Permit)

APPENDIX E

COMPREHENSIVE SITE COMPLIANCE EVALUATION REPORTS

March 2015

Others are available upon request

# STORM WATER POLLUTION PREVENTION

ANNUAL COMPREHENSIVE SITE COMPLIANCE

EVALUATION REPORT

WESTOVER AIR RESERVE BASE

MASSACHUSETTS

# Prepared for:

HEADQUARTERS, Air Force Reserve Command

HQ AFRC/CEVQ

255 Richard Ray Boulevard

Robins Air Force Base, Georgia 31098-6137

April 2015

1. 	INTRODUCTION

This Storm Water Pollution Prevention Plan (SWPPP) Annual Comprehensive Site Compliance Evaluation (CSCE) Report documents the findings of the SWPPP compliance inspection/evaluation conducted by Mr. Frank Doerneman and Mr. Jon Trombino of EA Engineering, Science, and Technology, Inc., PBC (EA) from 24-26 March 2015, at Westover Air Reserve Base (WARB) in Chicopee, Massachusetts. The CSCE and the report are an annual requirement of the WARB National Pollution Discharge Elimination System (NPDES) Storm Water Multi-Sector General Permit (MSGP).

The purpose of the annual CSCE is to determine the effectiveness of the Base’s SWPPP in controlling storm water pollution from industrial areas and to ensure the Base complies with its MSGP. This report reviews the Base’s industrial operations, discusses best management practices (BMPs) being successfully implemented, suggests additional BMPs to be implemented, and reports any instances of noncompliance with the permit. Each of the aspects of the SWPPP that are regulatory requirements were evaluated and determined to be in compliance unless otherwise noted in this report. Some of the BMPs suggested in the previous CSCEs were reevaluated during the 2015 CSCE and changes have been made to make the suggested BMPs more feasible to address the current conditions at WARB.

2.	STORM WATER REGULATION

a.	Storm Water Permit

The United States Environmental Protection Agency (US EPA) regulates the discharge of storm water from WARB under the Storm Water MSGP issued on June 4, 2015. The MSGP covers storm water discharge from Outfalls 001, 002, 003, 004, 005, 006, 007, 009, 011 and 011A.

b. 	MS4 Phase II Review

From the review of MS4 Phase II regulations it was determined that WARB is not regulated under Phase II regulations because it is not: (1) physically interconnected with any MS4 and (2) is designated for permit authorization by EPA. The facility is being considered for MS4 status under the upcoming new permit.  This will be review during the next CSCE.

3. 	CHANGES IN INDUSTRIAL OPERATIONS

The primary industrial activity at WARB has been, and will continue to be, the operation and maintenance of military equipment and 16 C-5 aircrafts. Changes that affect the potential for discharge of pollutants to surface waters since the 2014 CSCE include:

- Building 3400 – Has been removed from the IAP List – The Navy has vacated building 3400.  Building is temporarily occupied by Civil Engineering, but will become Communications.

- Hangar 7040- <redacted>, AMXS has approval for variable flow nozzle systems that will reduce the pressure and amount of glycol applied during deicing events.

4. 	CURRENT BMPs IMPLEMENTED

WARB has been proactive in implementing SWPPP BMPs since receiving coverage under its original NPDES permit, and has continued to be proactive in implementing SWPPP BMPs since receiving coverage under the NPDES Storm Water MSGP, active as of January 2001. Base personnel have also implemented many of the suggested BMPs recommended for pollution prevention, are aware of preventative maintenance practices, and promptly notify the appropriate personnel when a SWPPP concern needs to be addressed. Table 1 lists currently implemented and suggested BMPs as observed at the time of the CSCE.

6.	INDUSTRIAL AREAS OF CONCERN AND SUGGESTED BMPs

Several industrial areas at the Base have a potential to pollute storm water. These areas, along with suggested BMPs to correct the problems, are identified in Table 2 of this report. Implementation of BMPs, especially managing spills and leaks, will be maintained by conducting regular inspections at the IAPs buildings and parking areas as outlined in the SWPPP.

7. 	CONSTRUCTION ACTIVITIES

During the CSCE, the Base had two active construction projects.  The sites are described below and were evaluated for the potential of sediment and other contaminants to impact the storm drainage systems.

- James Street Entrance – This project involves the construction of a new entrance to provide access to the base from the east.  The site was visited and the SWPPP was inspected.  All BMPs were in place and properly maintained and all SWPPP records were up to date.

- Fuel Hydrant System – This project involves the construction of a new plane fuel hydrant loop system.  The project is being constructed in phases over the next two years and requires construction on the flight line apron and around the existing fuel pump houses and buildings. The site was visited and the SWPPP was inspected.  All BMPs were in place and properly maintained and all SWPPP records were up to date.

8. 	OUTFALL INSPECTION

Each outfall was visually inspected, including Outfalls 001, 002, 004, 006, 007, 009, and 011. The outfalls appeared to be in good condition with minimal intrusive vegetative growth or sediment blockage.   Due to snow melt and light rain, all of the outfalls had low to moderate flows, except for Outfall 009 which appeared to be frozen. All outfalls except for Outfalls 001, 002, and 011 had no odors, sheens or foams. Outfalls 001, 002, and 011 had very light white foam around the outfalls. EA performed a visual investigation of the outfall systems 001 and 002, and believed the cause was related to the fact the tops of the large open air OWSs were frozen over.  Since the tops were frozen the water surface elevation was not allowed to increase as flow increased which caused a surcharge on the inlet side of the OWSs.  With the surcharge and the ice causing the OWS to act as a closed pipe, it is believed that the glycol and detergents that had been captured by the OWS over the winter were being partially flushed out.  The water upstream of the OWS did not have an odor or foam.  Outfall 011’s light foam is believed to be caused by organic materials becoming agitated where it crosses under Perimeter Road.

9. 	INCIDENTS OF NONCOMPLIANCE

WARB has had no reportable incidents of noncompliance with its NPDES MSGP since the 2014 SWPPP review. Incidents of noncompliance relate to violations of the Multi-Sector General Permit requirements, thus violations of the Clean Water Act.

WARB did not have a reportable spill since the 2014 CSCE.

| Table 1. Current BMPs Implemented at Westover ARB as of March 2015 (Summarized from Appendix C – IAP forms) |  |  |  |  |
| --- | --- | --- | --- | --- |
| IAP LOCATION (Building Number) | Potential for Storm Water Contamination | ADDITIONALLY SUGGESTED BMP DURING CSCE | BMPs CURRENTLY IMPLEMENTED | BMPs EFFECTIVE YES/NO/NA |
| < MILITARY SHOP NAME REDACTED FROM PUBLIC ACCESS > | Low | - None. | - Absorbent material and booms are located inside the building by the loading dock doors. Additionally, a complete spill kit is located in the interior of the building. - Storage areas are bermed to provide secondary containment. | Yes |
|  | Low | - None. | - Prohibition of outdoor vehicle washing activities. - No fuel tankers allowed in building. | Yes |
|  | Low | - None. | - No floor drains in shops. - Prohibition of outdoor vehicle washing. - HAZMAT storage sheds provide a roof and secondary containment for paints. - Perform routine visual inspections for leaks in long-term vehicle and heavy equipment parking area; perform maintenance on vehicles and equipment prone to leaking | Yes |
|  | Low | - None. | - Perform routine visual inspections for leaks equipment in long-term parking area. - Bermed area that drains into a closed valved storm drain inlet. | Yes |
| < MILITARY SHOP NAME REDACTED FROM PUBLIC ACCESS > | Low | - None. | - OWS to be inspected (and cleaned as needed) along with Base schedule. - Hazmats stored in sheds and flammables lockers. - Perform routine visual inspections for leaks and spills from long-term parked equipment; perform maintenance on equipment prone to leaking. - Floors sloped inward toward main area. - Spill kits. - Spill pallets and drip pans. | Yes |
|  | Low | - None. | - The dome prevents precipitation from contacting sand pile. - Perform and document routine visual inspections for leaks from equipment parked in long-term parking area. - Utilize good housekeeping practices during salt loading/unloading operations. - Salt pile is covered by a tarpaulin. | Yes |
|  | Low | - Replacement of damaged poly secondary containment basin with brand new poly completed on 7 Apr 15. | - Spill kit inside and outside in parking area. - Drip Pans used on identified leaking vehicles. - Regular walk through inspections (records not kept). - Perform and document routine inspection of long term vehicles and equipment parking. | Yes |
| < MILITARY SHOP NAME REDACTED FROM PUBLIC ACCESS > | Low | - None. | - Small spill kit and absorbent material inside building. - Spill pallets are used to store containers/drums of HAZMATs while kept at Base Supply. - All parked generators and bowsers are drained of fluids and ready for shipment. - Perform routine visual inspections for leaks from generators and other equipment in long-term parking area. | Yes |
|  | Low | - None. | < FUEL SYSTEM INFO REDACTED FROM PUBLIC ACCESS > | Yes |
|  | Low | - None. | - < HANGAR BUILDING DETAILS REDACTED FROM PUBLIC ACCESS > - HAZMAT shed provides secondary containment for various HAZMATs. - Spill kits located in the hanger and in the HAZMAT shed. - Drainage Logs maintained in pump room. | Yes |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | Medium | - None | - < FUEL SYSTEMS DETAIL REDACTED FROM PUBLIC ACCESS > | Yes |
|  | Low | - None | - Spill kit next to propylene glycol fill stand (outside). - Perform routine inspections of long term parked vehicles. - Three spill kits located inside building. | Yes |
|  | Low | - Perform routine inspections for cracks, breaks, holes, and deterioration of secondary containment. | - <Infrastructure details redacted from public access> - Large spill kit at 7045 with spill booms. - Small “customer” spill kit present at 7046. - Bollards on either end of both fueling areas protect from vehicular damage. - Emergency stop for fuel pumps at each island and on the outside wall of Hangar 9. - Perform routine inspections for cracks, breaks, holes, and deterioration in the secondary containment for Bldg 7045. | Yes |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | Low | - None. | - < BUILDING DETAILS REDACTED > - Posted spill warning signs. - OWS Inspected quarterly (cleaned as needed). - Spill Kits throughout hangar. - Hazmats stored in sheds and flammables lockers. - Hangar openings have berms to contain spills inside hangar. - Perform routine visual inspections for leaks and spills from long-term parked equipment; perform maintenance on equipment prone to leaking. | Yes |
|  | Low | - None. | - Spill Kits inside all shop areas. - Flammables lockers and Hazmat sheds. - Spill Pallets. - All fluids from NDI are captured and recycled. - Routine inspections of long term vehicles for leaks and spills. | Yes |
|  | Low | - Clean up leak from small tractor mounted snow blower, and repair equipment was completed on 25 Mar 15. | - <Building details redacted from public access.> - Spill kit inside. - Drip pans used where necessary. - Routine inspection of vehicles for leaks, form 1806 kept as record of inspection (roads and grounds). - A spill boom is used to provide secondary containment at the waste oil accumulation point. | Yes |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | Low | - None. | - Vehicles inspected regularly. - Vehicles stored inside hangar during winter. - Absorbent pads and dry material for spills. | Yes |
|  | Low | - None. | - Washing occurs at an indoor approved wash rack that discharges to the sanitary sewer. - HAZMAT storage sheds provide a roof and secondary containment for various containers of HAZMATs. - Spill Kits on all vehicles. | Yes |
|  | Low | - None. | - Spill kits throughout the area (on each truck and in each pump house). - <Fuel systems details redacted from public access.> - Perform routine visual inspections of long term vehicles. | Yes |
|  | Low | - None. | - Containment tank serves the trench drains. - 50-gallon portable drip pans for use under refueler trucks. - Portable secondary containment devices are placed under leaky refueler trucks (visible leaks beyond berms). - Permanent concrete dike around drums/antifreeze/oil dispensing units. - Use of dry clean-up methods for fuel spills. | Yes |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | Low | - None. | - Firefighting chemicals are prohibited in fire training exercises. - Routine visual inspection of water conservation pond for sediment and vegetation accumulation. Clean vegetation out of the water conservation pond as necessary to maintain liner integrity. | Yes |
|  | Low | - None. | - Perform and document routine visual inspections for leaks from vehicles and equipment in long term parking area. | Yes |
|  | Low | - None. | - Secondary containment for fuel bowsers. - Spill kits brought with personnel during training exercises. - Empty 55-gallon drums are labeled “Empty”. | Yes |
|  | Medium | - None. | - <Infrastructure details redacted from public access.> - Complete and submit an Aircraft Glycol Monitoring Log to the CEV after each deicing event. - Routine visual inspections are performed for leaks from equipment that are parked on the apron. | Yes |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | Medium | - Perform routine visual inspections of snow removal equipment for leaks. - Use drip pans as needed on vehicles/equipment. - Perform regular inspections of temporary C5 planes that will be parked on ramp during fuel hydrant construction | - Concrete apron is graded to prevent storm water run-off. - The Fire Department is located on the flightline and can provide immediate HAZMAT response in the event of a fuel release. - <Aircraft info redacted from public access.> - Complete and submit an Aircraft Glycol Monitoring Log to the CEV after each deicing event. | Yes |
|  | Low | - Perform routine visual inspections during loading and unloading activities, and after large rain events. | - Grassed buffer area which storm water must flow across before entering Stony Brook. - Earthen berm to contain and direct runoff to the south and east. - Filter sock installed to the west for sediment and erosion control. - Contractors are to clean up tracked sediment from activities at the end of each day. | Yes |

| Table 2. Industrial Areas of Concern with Suggested BMPs |  |  |  |
| --- | --- | --- | --- |
| IAP LOCATION (Building Number) | AREA OF CONCERN | SUGGESTED BMPs | COMPLETION DATE |
| < MILITARY SHOP NAMES REDACTED FROM PUBLIC ACCESS > | - Potential spills from temporarily stored refueler trucks during Upgrade Hydrant Construction Project. | - Perform routine inspections of the area and the refueler trucks. | - Ongoing until construction is complete. |
|  | - Fluids from snow removal equipment could potentially contact storm water. - Spills from temporarily stored C5 planes during Upgrade Hydrant Construction Project. | - Perform routine visual inspections of snow removal equipment for leaks. - Use drip pans as needed. - <Aircraft info redacted from public access.> | - Ongoing |
|  | - Damaged secondary containment basin was replaced on 7 Apr 15. | - Replace damaged secondary containment basin for large generator stored in parking lot. | - 7 April 2015 |
|  | - Deicing fluid leaks from multiple deicing trucks was cleaned up on 8 Apr 15. | - Clean up multiple small leaks from deicing trucks inside of the hanger to prevent tracking of fluid outside. | - 8 April 2015 |
|  | - Recent leak from a small tractor mounted snow blower was cleaned up on 25 Mar 15. | - Clean up leak from small tractor mounted snow blower, and repair equipment. | - 25 March 2015 |

APPENDIX F

ROUTINE IAP INSPECTION LOG & OUTFALL VISUAL ASSESSMENTS

(Records located on the “R” drive under Natural Resources/Water/Stormwater/Inspections)

APPENDIX G

ANNUAL REVIEW UPDATE CHECKLIST AND

RECORD OF DOCUMENT REVIEW AND CHANGES

ANNUAL SWPPP UPDATE CHECKLIST

The following table provides a list of sections, figures, tables, and appendices in this plan that should be reviewed subsequent to any changes in Base structure or operations and upon completion of the annual comprehensive site compliance evaluation to evaluate the effect of any changes at the Base on storm water, and should be updated if necessary.

| SECTION | TITLE |
| --- | --- |
|  | SWPPP Cover Page |
|  | Certification Page |
| 1.1 | Regulatory Background |
| 1.2 | Physical Description and Mission of the Base |
| 1.4 | Storm Water Pollution Prevention Plan History |
| 3.2.1 | Watershed Identification and Characterization |
| 3.5.2 | Identification of Illicit Connections |
| 4.4.2 | Deicing BMPs |
| FIGURE | TITLE |
| 3 | Overall Facility Map Showing Cantonment Boundary, Watershed Boundaries, Surface waters, and Industrial Outfall Points |
| 4 | Overall Facility Map Showing Vehicle and Aircraft Maintenance Areas, Fuel Storage and Transfer Areas, O/W Separators, Storm Sewer Lines, and Past Spills and Leaks |
| TABLE | TITLE |
| 2 | Past Spills and Leaks |
| 5 | Westover ARB Summary of IAP and Non-Storm Water Survey |
| 10 | Approved Wash Racks at Westover ARB |
| 11 | BMPs That Need to be Routinely Performed |
| APPENDIX | TITLE |
| C | Westover ARB IAP Survey Forms |
| E | Comprehensive Site Compliance Evaluation Reports |
| G | Record of Document Review and Changes |
| H | Westover ARB Storm Water Monitoring Data |
| I | Photographic Record of Storm Water Outfalls |

RECORD OF DOCUMENT REVIEW AND CHANGES

| CHANGE NO. | ANNUAL REVIEW? (Y/N) | DATE | RESPONSIBLE PARTY | DESCRIPTION OF CHANGE |
| --- | --- | --- | --- | --- |
| ____ | N | 12/2/94 | SEA, Inc. | Final SWPPP Submitted |
| 001 | Y | 6/18/97 | SEA, Inc. | Annual SWPPP Review - Draft SWPPP Submitted |
| 002 | Y | 8/27/97 | SEA, Inc. | Annual SWPPP Review - Final SWPPP Submitted |
| 003 | Y | 7/16/99 | SEA, Inc. | Annual SWPPP Review – Draft Revised SWPPP Submitted |
| 004 | Y | 10/14/99 | SEA, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 005 | Y | 11/17/00 | E & E, Inc. | Annual SWPPP Review – Draft Revised SWPPP Submitted |
| 006 | Y | 1/24/01 | E & E, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 007 | Y | 12/6/01 | E & E, Inc. | Annual SWPPP Review – Draft Revised SWPPP Submitted |
| 008 | Y | 2/08/02 | E & E, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 009 | Y | 10/22/02 | E & E, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 010 | Y | 12/30/02 | E & E, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 011 | N | 1/24/03 | E & E, Inc. | Revisions to Final SWPPP Submitted 12/30/02 |
| 012 | Y | 11/04/03 | E & E, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 013 | Y | 13 Sep 04 | EA, Inc. | Annual SWPPP Review – Draft Revised SWPPP Submitted |
| 014 | Y | 05 Oct 04 | EA, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 015 | Y | 12 Aug 05 | EA, Inc. | Annual SWPPP Review – Draft Revised SWPPP Submitted |
| 016 | Y | 18 Nov 05 | EA, Inc. | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 017 | Y | 12 Sept 06 | EA, Inc | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 018 | Y | 27 Sept 07 | EA, Inc | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 019 | N | 22 Dec 08 | EA, Inc | 2008 MSGP SWPPP Review and Update- Final |
| 020 | Y | 4 Apr 09 | EA, Inc | 2009 MSGP SWPPP Review and Update- Draft |
| 021 | Y | 6 Aug 09 | EA, Inc | Annual SWPPP Review – Final Revised SWPPP Submitted |
| 022 | Y | 1 May 2010 | EA, Inc. | Draft Annual SWPPP Update 2010 |
| 023 | Y | 28 Oct 11 | WARB | Annual SWPPP Review & Revision |
| 024 | Y | 10 May 12 | EA, Inc. | Annual SWPPP Review & Revision |
| 025 | Y | March 2013 | EA, Inc. | Annual SWPPP Review & Revision |
| 026 | Y | April 2014 | EA, Inc. | Annual SWPPP Review & Revision |
| 027 | Y | April 2015 | EA, Inc. | Annual SWPPP Review & Revision |

APPENDIX H

WESTOVER ARB

STORM WATER MONITORING

(Records are located on the “R” drive under Natural Resources/Water/Stormwater/Monitoring Data)

APPENDIX I

PHOTOGRAPHIC RECORD OF STORM WATER OUTFALLS

< REDACTED FROM PUBLIC ACCESS >

Photograph 1 and 1a – Outfall 001 & OWS Discharge at Outfall 001

Photograph 2 and 2a – Outfall 002 and OWS discharge at Outfall 2

Photograph 3 – Outfall 003 (2013 Photo)

Photograph 4 – Outfall 004

Photograph 5 – Outfall 005 (2013 Photo)

Photograph 6 – Outfall 006

Photograph 7 – Outfall 007

Photograph 8 – Outfall 009

Photograph 9 – Outfall 011

Photograph 10 – Outfall 011

APPENDIX J

LETTER OF DESIGNATION – AUTHORIZING THE SIGNING OF SWPPP’s

APPENDIX K

DEICING CHEMICALS MONTHLY QUANTITIES

(Records located on the “L” drive under AMXS/Shared Data/AMXS De-ice)

DEICING CHEMICALS MONTHLY QUANTITIES - FY2011

|  |  |  |  | TO OUTFALL #1 (Wetland) |  | TO OUTFALL #2 (gallons) |  | TO OUTFALL #6-or whichever is towards the west (gallons) |  | TO OUTFALL #3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Ac | Vehic | Locatio | Deicin | Wat | Deicing | Wat | Deicin | Wat | Deicing Fluid Only | Water Only | Remarks (time) |

APPENDIX L

AFFF PROTOCAL AND RELATED REGULATIONS

AFFF Readiness Protocol

The 439th MSG/CEF, Fire Department, is required to have AFFF available for immediate discharge to control and extinguish hydrocarbon spills and/or fires.  To show that the resultant foam would meet NFPA and Air Force Standards, operational tests of vehicle and building fire suppression systems must be conducted at regular intervals.

The following is a brief synopsis of system testing and inspection conducted by the Fire Department:

- The Aircraft Rescue and Fire Fighting (ARFF) Vehicles carry AFFF.  AFFF is discharged from hoses, nozzles and turrets directly into the “Fire Training Pond”, which is an engineered pit with liner from which flow is directed to the sanitary system.

- Facility 7040, Pull Through Hangar. System testing is conducted every two years; foam is sprayed into a 30,000 gallon wastewater holding tank that discharges into the sanitary system.

- Facility 7705 Pump House #3 and Facility 7000 West Side.  System testing is conducted every two years, whereby hoses are connected to foam risers within the building and the foam is discharged onto a nearby grassy field, away from storm drains.  Testing is typically done during dry weather conditions.  On rare occasion, system testing at Facility 7040 also results in discharging foam to nearby grassy area.

CERCLA and EPCRA Requirements Relative to AFFF

CERCLA 103 and EPCRA 304 notification requirements no longer apply to releases of AFFF containing ethanol, 2-(2-butoxyethoxy), unless the AFFF released contains another listed CERCLA hazardous substance found at 40 CFR 302.4 or extremely hazardous substance found at 40 CFR Part 355 Appendix A; the type of AFFF used at Westover does not contain any of these listed substances.  Releases of AFFF containing only chemicals within the glycol ethers category no longer require reporting to the National Response Center pursuant to CERCLA 103(a) or the State Emergency Response Commission and Local Emergency Planning Committee pursuant to EPCRA 304.  Owner/operators can still be held liable under CERCLA for clean-up costs or damages caused by a release of AFFF containing a glycol ether, even though the release itself is not reportable (60 FR 30926, 30933; June 12, 1995).

Although there are no laws regulating the use of Ansulite 3 percent AFFF, which is the type used on Base, the EPA is concerned because the final degradation product is unknown.  There is ongoing research to determine if AFFF degrades down to a compound called perfluorooctanoic acid (PFOA).  Although EPA concludes that PFOA “is persistent” and “may bioaccumulate …in the same manner as perfluorooctanesulfonyl fluoride (PFOS) does,” it does not officially classify PFOA as PBT, persistent-bioaccumulative-and toxic.  EPA will continue to evaluate new data on the environmental impacts of PFOA and is likely to update this hazard assessment in the future.  (Fire Fighting Foam Coalition, “AFFF Update” May 2002)

APPENDIX M

SUBSTANTIALLY IDENTICAL OUTFALL ASSESSMENT <Outfall details and location info redacted from public access due to security concerns.>

&

RUN-OFF COEFFICIENT CALCULATIONS

APPENDIX N

2013 E. COLI ASSESSMENT REPORT

MMaaterials that could be discharged to the drainage areas of the outfalls as a result of industrial activity at each IAP is presented in Appendix C. To satisfy the requirement, that types of discharge contained in drainage areas be displayed on a figure, IAPs have been designated as a vehicle or aircraft maintenance area, a fuel transfer and storage area, or other industrial area.